Speech Thing Update Notice

Software Release Version 3.0

April, 1992

NOTE:  You can print out this file by typing

PRINT README.DOC at the DOS prompt.

MANUAL CORRECTIONS.

Page 30, remove steps 3,4, & 5.

Page 43, in the section titled "SAY Switch Commands", the

/Q switch has the incorrect description.  /Q is for

"Quiet mode" which suppresses any message that is printed

to the screen during playback.  This is desirable for

batch processing or DOS Shell commands when you call this

from within a program.  Using /Q is equivalent to using

">NUL" on the command line.

MULTIPLE TEXT-TO-SPEECH PROGRAMS

Your Speech Thing has been refined over the last several

years with state-of-the-art software.  This version

currently contains the latest generation of text-to-

speech software, version 4.  To maintain compatibility

with previous Speech Software, we've included both

versions 2 and 3, SPEECHV2 and SPEECHV3, respectively.

These previous versions are ideal for adding text-to-

speech into your own programs.

There are several text files that describe how to use and

program with utilities:

STDRIVER.DOC   Documentation on how to install and

program the Speech Thing DOS device

driver, STDRIVER.SYS.

STDEMO.DOC     How to use the GWBASIC program, STDEMO.BAS

PHONETIC.DOC   Information on how to use phonetic

commands.

README.DOC     This file.

Any of these files can by viewed using the DOS TYPE

command.  Example:  TYPE STDEMO.DOC |MORE.

Use the DOS PRINT command to print the file.  Or you can

load these ASCII text files into your favorite text or

word processor.

IBMPLAY.

IBMPLAY.EXE is a utility that will play back speech

encoded using the CVSD technique (Continuously Variable

Slope Delta-modulation) through your Speech Thing.  CVSD

is used in many software titles that specify the IBM

Speech Feature card. Examples are "Writing to Read" and

"Comptons Multimedia Encyclopedia".

To load the driver, type IBMPLAY /P LPT1 at the DOS

prompt before you run the application program.  To test

if the driver is working correctly, we've included a CVSD

encoded sound file called MCARTHUR.SPK.  Once IBMPLAY is

loaded, type CVSD MCARTHUR.SPK and you'll hear General

Douglas McArthur's farewell speech to Congress, complete

with static and applause.

You can remove the driver from memory by typing:

IBMPLAY /Q.

You can list the options available to you by typing:

IBMPLAY /?

For your convenience, we've set up the Launcher program

to load the driver and speak the file.  NOTE:  an 6MHZ AT

or faster PC is required for IBMPLAY.

CONVERT.

CONVERT.EXE is a utility to convert between sound

formats. The instructions are in a file called

CONVERT.DOC.  Experiment by compressing some of the 8-bit

files (.V8) down to 3 bit (.V3S) or even 2-bit (.V2S).

I think you'll find our compression technology to be one

of the best in the industry.

COVOX ELECTRONIC BULLETIN BOARD.

If you have a MODEM, you can log into our BBS and

download all sorts of files for use with the Speech

Thing.  This includes more sound clips, ModPlay Modules,

and developer tools.  For complete instructions on using

our BBS, to the text file BBS.DOC.