Documentation for

The Speech Thing Device Driver

STDRIVER.SYS Version 2.73

Copyright 1988-92 COVOX INC.

April, 1992

INTRODUCTION

The Speech Thing device driver supports synthesized speech

generated by the Smooth Talker programs from 'First Byte'

(SPEECHV2.EXE and SPEECHV3.EXE).  The device driver DOES NOT

support the Version 4 text-to-speech program as used in the main

program.

NOTE:  The device driver program is generally not

recommended except in the following cases:

(1) Third party software that specifically requests

that you install STDRIVER.SYS, or

(2)  You wish to utilize the progamming features of the

driver for your own custom programs or for general

interest.

The device driver program, "STDRIVER.SYS", will allow the

Speech Thing to be employed in a wide variety of languages and

operating environments.  A device driver can also be accessed

directly from DOS in much the same way as disk files.  For

instance, ASCII files can be sent to the device driver using

DOS's 'TYPE' or 'COPY' command and Smooth Talker will then

promptly speak them back.

The first section describes how to install the STDRIVER.SYS

driver program followed by the operation and functions of the

driver as it pertains to text-to-speech.

INSTALLING STDRIVER.SYS

The device driver STDRIVER.SYS will work with any PC or MS

DOS version 3.0 or higher.  It can be installed by including it

in a 'CONFIG.SYS' file which must reside in the root directory of

the drive that is used to boot the system.  If a CONFIG.SYS file

already exists, then just add the STDRIVER.SYS driver program to

it by including the following line:

DEVICE = \COVOX\STHING\STDRIVER.SYS /SV3

This will cause the STDRIVER.SYS device driver to be

installed the next time the system is booted up.

The /SV3 is an optional parameter which tells the driver

that it is using SPEECHV3.  The default is SPEECHV2.EXE.

DEVICE = \COVOX\STHING\STDRIVER.SYS /SV3 /DAC=559

This will cause the driver to use SPEECHV3.EXE and output

the speech to port number 559 (or 22F in hex) which happens to be

the default port setting for Sound Master II and Voice Master

(VM0).

A list of port used by COVOX is given as follows:

Speech Thing

LPT1      DEFAULT no /DAC= needed

LPT2      632 usually

LPT2      888 if monochrome card present

Voice Master Key and Sound Master II

VM0       559

VM1       591

VM2       655

VM3       719

In general, the command line format for STDRIVER is as follows:

DEVICE = \COVOX\STHING\STDRIVER.SYS /P/P/P/etc.

^ a mandatory space

Where the 'P' parameter represents any legal STDRIVER.SYS device

command.  For example

DEVICE = \COVOX\STHING\STDRIVER.SYS /PHO/TTD

Data sent to the driver would be interpreted as phonetic

strings (PHO) and the string would not be echoed to the screen

when spoken (TTD).  These settings will remain in effect until a

command is sent to the driver which changes one or more of them.

If no parameters are entered, the device driver will install

itself in the default mode which would be equivalent to the

following:

DEVICE = \COVOX\STHING\STDRIVER.SYS /ENG/CRE/TTE/TON=0

Keep in mind that the parsing routine will not accept an

illegal parameter and if one is found then parsing will stop

where the error occurs and default parameters will be used for

the remaining parameters.  Of course, the driver will notify you

of any illegal parameters when it installs itself at boot- up

time, but it will not specify exactly where the error occurred.

You must determine this yourself.

NOTE: The Speech Thing device driver will work correctly only

with DOS versions 3.0 or higher.

Also, don't forget that STDRIVER.SYS requires either

SPEECHV2.EXE (for slow PC's like XT's) or SPEECHV3.EXE (for AT's

or faster) to be loaded in memory before any strings can be sent

to the driver.  Probably the best way to do this is to include

one of the two programs in an 'AUTOEXEC.BAT' file and then it

will automatically be loaded into memory upon boot-up. After you

are finished, both SPEECHV3.EXE and SPEECHV2.EXE can be removed

from memory by typing the name of the program followed by a /Q.

For example,

SPEECHV3.EXE /Q

removes the text-to-speech program from memory.  However, the

driver program, STDRIVER.SYS, still remains in memory.  The only

way to remove STDRIVER.SYS is to not include it in your

CONFIG.SYS file.

--- Device Driver IOCTL Commands ---

The driver supports three distinct functions, WRITE, READ

and IOCTL.  Input Output ConTroL or IOCTL is function 44H of DOS

interrupt 21H and is used to send commands to the Speech Thing

device driver.  Some languages like GW-BASIC and QuickBasic

specifically support the IOCTL function and commands can easily

be passed to the driver in the form of a three character ASCII

string.  In cases where the IOCTL function is not specifically

supported, such as with Assembly language, Turbo PASCAL, or

QuickC, then commands must be sent to the driver using the DOS

interrupt directly.  Some of the IOCTL commands are used to pass

a numerical value to the driver program.  This is done by

immediately following the three character command with an equal

sign, '=', and then the decimal number for the value to pass.

For example, sending the IOCTL string 'SPE=8' would tell the

driver to set the playback speed to a value of eight.

IMPORTANT: When sending a parameter with an IOCTL command, this

exact format, within the quotes, must be adhered to: 'CMD=XXXX'.

Where CMD is the IOCTL command, which can be either upper or

lower case letters, followed immediately by the equal sign and

finally the decimal value.  Note that no space characters or

commas are allowed.

SMOOTH TALKER MODE

This mode of operation causes the device driver to pass data

from DOS or the application program to the Speech Thing after the

data has been translated by Smooth Talker.  Therefore it is

mandatory that either SPEECHV2.EXE or SPEECHV3.EXE be resident in

memory when using this mode.  When in this mode, the driver reads

and writes data one byte or character at a time which means that

an intermediate buffer between the driver and DOS, or an

application program, is not required.  Data bytes are passed to

and from the driver as a stream of ASCII characters representing

either English or phonetic structures. Data is passed in several

ways, from DOS by using its 'COPY' or 'TYPE' command or from an

application program using a WRITE or READ function.  The ability

to Send a string of characters directly from DOS to the driver is

one advantage that STDRIVER.SYS has over the STALK program.  On

the other hand, STDRIVER.SYS does not support any of Smooth

Talker's dictionary functions.

Smooth Talker can continuously playback an English or

phonetic string of up to 255 characters before it must pause to

reset and read in the next string.  If this pause occurs in the

middle of a sentence, then the speech will sound discontinuous.

For this reason, the driver program breaks up the string of input

characters into grammatical sentences with natural pauses and

then passes each sentence on to Smooth Talker for interpretation.

A natural pause is considered by the driver to be a period,

colon, question or exclamation mark.  After Smooth Talker

interprets the sentence, it sends the result to the Speech Thing

for playback and then resets itself to receive another 255

characters.  If the driver is sent a string that cannot be

divided into sentences, then it will send the maximum of 255

characters to Smooth Talker and then pause.  There is one

exception to the above which is the 'carriage return' character.

The driver can optionally be set to either ignore or acknowledge

carriage returns.  If acknowledged, then a carriage return

character has exactly the same effect as a period or other

sentence terminator.  Causing smooth talker to speak directly

from the DOS prompt can be done using one of three DOS commands

as follows:

A>COPY SAMPLE ST

or

A>TYPE SAMPLE > ST

or

A>ECHO SAMPLE >ST

These methods have the same effect and will cause Smooth

Talker to playback the file.  Also the 'SHELL' command could be

used with equal effect:

SHELL "COPY SAMPLE ST"

or

SHELL "TYPE SAMPLE > ST"

The above file, "SAMPLE", could be any ASCII text file of

any length and can be created with a word processor, EDLIN, or

directly from the DOS prompt.  Here is an example of creating a

file from the DOS prompt:

A>COPY CON: SAMPLE

This is a sample text file.

^Z

Enter the above text exactly as shown and press ENTER after

each line.  The CONTROL-Z character on the last line is the 'end

of file' marker and instructs DOS to create the file.

IOCTL Commands in Smooth Talker Mode

ENG  - This instructs the driver that ASCII characters passed

to it for playback will be sent as English strings.  After

issuing this command, all subsequent WRITEs by DOS or the

application program will cause smooth talker to interpret

the string as English and speak it.  Once an 'ENG' command

is sent, the driver will continue to interpret strings as

English until specifically told to do otherwise by issuing

the 'PHO' command.  If a phonetic string is sent to the

driver while it is in English mode, then it will attempt

to speak the string as English.  Also, the default mode

for the driver is English and the 'ENG' command need not

be sent if the driver was not previously in phonetic mode.

PHO  - Same as for 'ENG' above, but instructs the driver that

phonetic strings will be passed instead.

XEP  - This instructs the driver that the next string sent to

it is to be converted from English to phonetics.

Immediately after issuing this command the application

program should send an English string via a WRITE command

to the driver, then the phonetic result can be obtained by

sending the driver a READ command.  Once the 'XEP' command

has been issued, first a WRITE, then a subsequent READ

must be performed before the driver will return to its

normal I/O state.  Also, the driver expects that the

string written to it will be an English string, even if

the driver is currently in phonetics mode.  There is no

need to first issue an 'ENG' command before sending the

English string for the 'XEP' function.  After issuing a

READ to get the phonetic result, the driver will

automatically be reset to its previous English or phonetic

mode.

TTE  - This enables the Teletype mode which causes string data

passed to the driver to be echoed onto the screen.

TTD  - This disables the Teletype mode.

CRE  - Causes the driver to acknowledge carriage returns.  This

feature should be enabled in applications where it is

desired to have Smooth Talker speak the displayed string

when an ENTER or RETURN key is pressed.

CRD  - Causes the driver to ignore carriage returns.  This is

useful when Smooth Talker is being used to speak back

ASCII text files where carriage returns occur after every

screen line and cause unwanted pauses.

FLU  - Flushes out the 255 byte string buffer that the device

driver uses for passing strings to the SPEECHV2 or

SPEECHV3 program.  Sometimes it is necessary to clear out

an English or phonetic string that was previously sent to

the driver but never spoken by Smooth Talker.  This

situation can occur when carriage returns have been

disabled and a string without a sentence terminator (a

period, colon, etc.) is sent to the driver.

DAC  - Enable an alternate port address for output.  By default

the driver uses the port number at LPT1 (0:408 & 0:409)

for sending data to a DAC (Digital to Analog Converter).

This command allows an alternate port address to be

specified as the DAC port.  Range: DAC=0 to DAC=65535.

TON  - Sets the tone that smooth talker speaks with.  The tone

control will only work the SPEECHV3 program and has no

effect if changed for SPEECHV2.  Range: TON=0 for low tone

or TON=1 for high tone.

SPE  - Sets the speed of playback.  Range: SPE=0 to SPE=9.

PIT  - Sets the pitch of playback.  Range: PIT=0 to PIT=9.

VOL  - Sets the volume of playback.  Range: VOL=0 to VOL=8.

--- Programming in Smooth Talker Mode ---

Besides the IOCTL function, two other main I/O functions are

also supported by the device driver.  They are READ and WRITE

which correspond, respectively, to functions 3FH and 40H of DOS

interrupt 21H.  In 'Smooth Talker Mode', data is passed to and

from the driver as a string of ASCII characters whenever a WRITE

or READ is performed.  The maximum length of a string that is

written to the driver will depend on the limitations of the

language being used.  For example, GW-BASIC only allows strings

with a maximum length of 255 characters whereas QuickBasic

strings can be up to 32,767 characters in length. On the other

hand, the READ function always returns a string of 255 characters

or less.  In this present implementation of the driver program,

the READ function is only used in association with the 'eXchange

English for Phonetics' or 'XEP' command.  In order to convert an

English string to a phonetic string, the 'XEP' command is sent

using the IOCTL function followed by sending an English string

with a WRITE command.  Finally, the phonetic result is obtained

by using the READ function. Because the 'XEP' command employs all

three of the available driver I/O functions, it will be used

below as the primary example for passing string data to and from

the driver.

Example A

------------------------ GW-BASIC -----------------------

100 ON ERROR GOTO 430  'Handle any I/O errors.

110 CMD$="XEP" 'Assign the device command to a string.

130 ENGLISH$="This is a sample string." 'Assign the string to

write.

150 GOSUB 180  'Go do the conversion.

160 END

170 '

180 'ENGLISH TO PHONETICS

190 GOSUB 240  'Send the command.

200 GOSUB 300  'Send the English string.

210 GOSUB 360  'Get phonetic string.

220 RETURN

230 '

240 'DEVICE IOCTL

250 OPEN "ST" FOR OUTPUT AS #1 'First open device for IOCTL

write access.

260 IOCTL #1,CMD$      'Send the command to the driver.

270 CLOSE #1   'Close device for IOCTL write access.

280 RETURN

290 '

300 'DEVICE WRITE

310 OPEN "ST" FOR OUTPUT AS #1 'Open device for write access.

320 PRINT #1,ENGLISH$  'Send the English string to the driver.

330 CLOSE #1   'Close device for write access

340 RETURN

350 '

360 'DEVICE READ

370 OPEN "ST" FOR INPUT AS #1  'Open device for read access.

380 INPUT #1,PHONETIC$ 'Read the phonetic conversion from the

390 PRINT PHONETIC$    'driver and display it to the screen.

400 CLOSE #1

410 RETURN     'Exit from routine and return to caller.

420 '

430 PRINT "AN I/O ERROR HAS OCCURRED"

440 GOTO 160

---------------------------------------------------------

Example B

------------------------ QuickBASIC ---------------------

ON ERROR GOTO PRINT.MESSAGE  'Handle any I/O errors.

CMD$="XEP"   'Assign the device command to a string.

ENGLISH$="This is a sample string."  'Assign the English string

to translate.

CALL ENGLISH.TO.PHONETICS    'Go do conversion.

TERMINATE: END       'End this program.

SUB ENGLISH.TO.PHONETICS STATIC

CALL DEVICE.IOCTL   'Send the command.

CALL DEVICE.WRITE   'Send the English string.

CALL DEVICE.READ    'Get and print the phonetic equivalent.

END SUB

SUB DEVICE.IOCTL STATIC

SHARED CMD$ '(allow access to an external variable)

OPEN "ST" FOR OUTPUT AS #1  'First open device for I/O-WRITE

access.

IOCTL #1,CMD$       'Send the command to the driver.

CLOSE #1    'Close the device for I/O-write access.

END SUB

SUB DEVICE.WRITE STATIC

SHARED ENGLISH$     '(allow access to an external variable)

OPEN "ST" FOR OUTPUT AS #1  'Open the device for a WRITE

access.

PRINT #1,ENGLISH$   'Send the English string to the driver.

CLOSE #1    'Close the device for write access

END SUB

SUB DEVICE.READ STATIC

OPEN "ST" FOR INPUT AS #1   'and re-open it for a READ

access.

INPUT #1,PHONETIC$  'Read the phonetic conversion from the

PRINT:PRINT PHONETIC$       'driver and display it to the

screen.

CLOSE#1     'Close read access and release file handle.

END SUB

PRINT.MESSAGE: 'If an error, then print message and

PRINT "AN I/O ERROR HAS OCCURRED"

RESUME TERMINATE       'terminate program.

-------------------------------------------------------

Example C

------------------- Assembly Language -------------------

CSEG    SEGMENT 'CODE'

ASSUME  CS:CSEG,SS:CSEG,DS:CSEG,ES:CSEG

ORG     100H

ASM_XEP_SAMPLE: CALL    ENGLISH_TO_PHON

RET     ;Return to DOS or calling routine.

CMD$    DB      3,'XEP'

ENGLISH$DB      25,'This is a sample string.',CR

;EQUATES AND VARIABLES

READ    EQU     0

WRITE   EQU     1

CR      EQU     0DH    ;Carriage return character.

DEVICE_NAME     DB      'ST',0 ;ASCIIZ file specification for

;the driver.

HANDLE  DW      0      ;File handle will be saved here.

WRITE_STRING    DW      0      ;This holds the address of the

:string to send.

WRITE_COUNT     DW      0      ;Total number of characters in

the above string.

READ_STRING     DB      100H DUP(0) ;Buffer for the returned

string.

READ_COUNT      DB      0   ;Length of the returned string.

ERR_MSG DB      'An error has occurred!',07H,CR,'$'

;ENGLISH TO PHONETICS PROCEDURE

ENGLISH_TO_PHON PROC    NEAR

;Open the driver for a write access.

MOV     AL,WRITE

CALL    DEVICE_OPEN

JC      ERROR

;Send a command string to the driver.

CALL    DEVICE_IOCTL

JC      ERROR

;Send the English string to be translated.

MOV     AL,ENGLISH$      ;Get number of characters in string

MOV     AH,0

MOV     WRITE_COUNT,AX   ;and save it for write.

MOV     AX,OFFSET ENGLISH$+1 ;Get address of the English

MOV     WRITE_STRING,AX  ;string and save it also.

CALL    DEVICE_WRITE     ;Go translate English to phonetics.

JC      ERROR

;Terminate the write access to the driver.

CALL    DEVICE_CLOSE

JC      ERROR

;Open the driver for a read access.

MOV     AL,READ

CALL    DEVICE_OPEN

JC      ERROR

;Get the phonetic translation from the driver.

CALL    DEVICE_READ

JC      ERROR

MOV     DX,OFFSET READ_STRING  ;Get address of phonetic string.

MOV     BL,READ_COUNT   ;Get length of string (always < 255).

MOV     BH,0

MOV     READ_STRING[BX],'$' ;Add a DOS string terminator and

CALL    PRINT       ;and go display the string.

;Terminate the read access to the driver and release the handle.

CALL    DEVICE_CLOSE

JC      ERROR

JMP     SHORT E_TO_P_DONE

;If an error occurs then notify user.

ERROR:  MOV     DX,OFFSET ERR_MSG

CALL    PRINT

CALL    DEVICE_CLOSE

E_TO_P_DONE:    RET     ;Exit this procedure.

ENGLISH_TO_PHON ENDP

;INPUT/OUTPUT PROCEDURES

DEVICE_OPEN     PROC    NEAR       ;Open access to the 'ST'

device.

MOV     AH,3DH

MOV     DX,OFFSET DEVICE_NAME

INT     21H

MOV     HANDLE,AX  ;Get and save handle for further access.

RET

DEVICE_OPEN     ENDP

DEVICE_CLOSE    PROC    NEAR       ;Close access to the 'ST'

device.

MOV     AH,3EH

MOV     BX,HANDLE

INT     21H

RET

DEVICE_CLOSE    ENDP

DEVICE_IOCTL    PROC    NEAR       ;Write an IOCTL command to

the 'ST' device.

MOV     AH,44H     ;DOS's 'DEVICE DRIVER CONTROL' function.

MOV     AL,3       ;Indicate a 'write command' sub-function.

MOV     BX,HANDLE

MOV     CL,CMD$    ;Get length of command to driver.

MOV     CH,0

MOV     DX,OFFSET CMD$+1 ;Get address of command

INT     21H;string and send it to driver.

RET

DEVICE_IOCTL    ENDP

DEVICE_WRITE    PROC    NEAR       ;Write a string to the 'ST'

device.

MOV     AH,40H     ;DOS's 'WRITE to DEVICE' function.

MOV     BX,HANDLE

MOV     CX,WRITE_COUNT ;Tell driver number of characters in

MOV     DX,WRITE_STRING ;string and address of string.

INT     21H;Send the string to the driver.

RET

DEVICE_WRITE    ENDP

DEVICE_READ     PROC    NEAR       ;Read a string from the 'ST'

device.

MOV     AH,3FH     ;DOS's 'READ from DEVICE' function.

MOV     BX,HANDLE

MOV     CX,0FFH    ;Maximum number of characters to read.

MOV     DX,OFFSET READ_STRING  ;Address of string buffer.

INT     21H;Tell driver to return a string.

MOV     READ_COUNT,AL ;Save length of string returned.

RET

DEVICE_READ     ENDP

PRINT   PROC    NEAR       ;Print a string to the screen.

MOV     AH,09H

INT     21H

RET

PRINT   ENDP

CSEG    ENDS

END     ASM_XEP_SAMPLE

-----------------------------------------------------

Example D

-------------------- Turbo PASCAL ---------------------

program pascal_XEP_sample;

type

data_str = string[16];

long_str = string[255];

regs = record

ax,bx,cx,dx,bp,si,di,ds,es,flags: integer;

end;

var

device, cmd_str: data_str;

english_str, phonetic_str: long_str;

result: regs;

handle, carry: integer;

work_var, read_len: byte;

phonetic_array: array[1..256] of char;

const

write = 1;

read = 2;

procedure device_open(access : byte);

begin

device := 'ST' + chr(0);

with result do

begin

ax := $3D00 + access;

dx := ofs(device)+1;

ds := seg(device);

end;

msdos(result);       {Open device to get handle}

with result do

begin

carry := flags and 1;

handle := ax;

end;

end;

procedure device_close;

begin

with result do

begin

ax := $3E00;

bx := handle;

end;

msdos(result);       {Close device}

with result do

begin

carry := flags and 1;

end;

end;

procedure device_ioctl;

begin

with result do

begin

ax := $4403;

bx := handle;

cx := length(cmd_str);

dx := ofs(cmd_str)+1;

ds := seg(cmd_str);

end;

msdos(result);       {Send I/O command}

with result do

begin

carry := flags and 1;

end;

end;

procedure device_write;

begin

with result do

begin

ax := $4000;

bx := handle;

cx := length(english_str);

dx := ofs(english_str)+1;

ds := seg(english_str);

end;

msdos(result);       {Write string to device}

with result do

begin

carry := flags and 1;

end;

end;

procedure device_read;

begin

with result do

begin

ax := $3F00;

bx := handle;

cx := $00FF;

dx := ofs(phonetic_array);

ds := seg(phonetic_array);

end;

msdos(result);       {Read string from device}

with result do

begin

carry := flags and 1;

read_len := ax;

end;

work_var := read_len;

phonetic_str := '';

while read_len > 0 do{Read the result into an array}

begin      {PASCAL programmers: Is their another way?}

read_len := read_len - 1;

phonetic_str := phonetic_str + phonetic_array[work_var -

read_len];

end;

end;

procedure print_error;

begin

writeln('An error has occurred!');

end;

procedure english_to_phon;

begin

device_open(write);

if carry=0 then

begin

device_ioctl;

if carry=0 then

begin

device_write;

if carry=0 then

begin

device_close;

device_open(read);

if carry=0 then

begin

device_read;

if carry=0 then

begin

writeln(phonetic_str);

device_close;

end;

end;

end;

end;

end;

if carry=1 then

print_error;

device_close;

end;

begin

cmd_str := 'XEP';      {IOCTL command to send to driver}

english_str := 'This is a sample string.' + chr(13);  {String

to convert}

english_to_phon;      {Go convert string to phonetics}

end.

---------------------------------------------------------

Example E

-------------------------- QuickC -----------------------

/*

Sample program for using voice master device driver with

the C programming language.

Using: MicroSoft Quick C ver 1.0 compiled with EXE option set.

*/

#include <stdio.h>

#include <dos.h>

#define READ  0x00

#define WRITE  0x01

char far *english = "This is a sample string.\r";

char phonetic[256];

char far *cmd[] = {"SV2","XEP",""};

/* procedure area */

int device_open(int *handle,int mode)

/* open ST device */

{

union REGS regs;

struct SREGS segregs;

static char far *file_name = "ST";

regs.h.ah = 0x3D;

regs.h.al = (unsigned char) mode;

segregs.ds = FP_SEG(file_name);

regs.x.dx = FP_OFF(file_name);

intdosx(&regs,&regs,&segregs);

if(regs.x.cflag) return(-1);

*handle =  regs.x.ax;

return(0);

}

int device_close(int handle)

/* Close access to the 'ST' device */

{

union REGS regs;

regs.x.bx = handle;

regs.x.ax = 0x3E00;

intdos(&regs,&regs);

return(0);

}

int device_ioctl(char far *cmd_string)

/* Write an IOCTL command to the ST device */

{

union REGS regs;

struct SREGS segregs;

int handle;

if(device_open(&handle,WRITE)) return(-1);

regs.x.ax = 0x4403;

regs.x.bx = handle;

regs.x.cx = strlen(cmd_string);

segregs.ds = FP_SEG(cmd_string);

regs.x.dx = FP_OFF(cmd_string);

intdosx(&regs,&regs,&segregs);

device_close(handle);

if(regs.x.cflag) return(-1);

return(0);

}

int device_write(char far *string)

/* Write string to the 'ST' device */

{

union REGS regs;

struct SREGS segregs;

int handle;

if(device_open(&handle,WRITE)) return(-1);

regs.x.ax = 0x4000;

regs.x.bx = handle;

regs.x.cx = strlen(string);

regs.x.dx =  FP_OFF(string);

segregs.ds = FP_SEG(string);

intdosx(&regs,&regs,&segregs);

if(regs.x.cflag) return(-1);

device_close(handle);

return(0);

}

int device_read(char far *string,int max_length)

/* Read a string from the 'ST' device */

{

union REGS regs;

struct SREGS segregs;

int handle;

if(device_open(&handle,READ)) return(-1);

regs.x.ax = 0x3F00;

regs.x.bx = handle;

regs.x.cx = max_length;

regs.x.dx =  FP_OFF(string);

segregs.ds = FP_SEG(string);

intdosx(&regs,&regs,&segregs);

device_close(handle);

if(regs.x.cflag) return(-1);

return(0);

}

main ()

{

int i;

i = 0;

/* keep sending command until a zero length string */

while(strlen(cmd[i])){

if(device_ioctl(cmd[i++])){

printf("An I/O command error has occurred");

exit(-1);

}

}

if(device_write(english)){

printf("A device write error");

exit(-1);

}

if(device_read((char far *)phonetic,sizeof(phonetic))){

printf("A device read error");

exit(-1);

}

printf("\n%s\n",phonetic);

return(0);

}

-------------------------------------------------------

The above programming examples contain all the procedures

necessary for sending any command or string of data to the driver

program.  In order to send a different command to the driver,

simply change the string assigned to the variable 'CMD$'.

Likewise, 'ENGLISH$' can also be changed as desired. In the

program example that follows, procedures and variables that have

already been defined will be assumed.  Here is a QuickBASIC

example that sets the playback speed to '8' and causes Smooth

Talker to speak the greeting, 'Hello, how are doing?'.  Simply

make the following changes to the above QuickBASIC program

(EXAMPLE B):

Example F

---------------------------------------------------------

CMD$="SPE=8" 'The IOCTL command to set playback speed.

ENGLISH$="Hello, how are you doing?" 'Assign the English string

to speak.

CALL SPEAK.ENGLISH   'Go speak the greeting.

SUB SPEAK.ENGLISH

CALL DEVICE.IOCTL 'Send the speed command.

CALL DEVICE.WRITE 'Send the English string.

END SUB

--------------------------------------------------------

Because the programming examples given for the other

languages also use the string variables 'CMD$' and 'ENGLISH$' and

contain I/O procedures with the same labels, the above logic can

be applied to them as well.  If this is not yet apparent, then go

back and study the examples very carefully.

NOTE: All the program examples given in this documentation have

been checked for accuracy and should be executable.  We hope that

they will provide the programmer with adequate information for

interfacing with our device driver.  However, some of the

examples could probably be written more effectively.  So, if you

have written a program (or re-written one of the above examples)

that you believe better demonstrates the use of the STDRIVER.SYS

device driver, then send us a copy and if we like it we will

include it in this documentation along with full credit to you

for your efforts.  We would also be interested in seeing examples

from other languages and application programs, like ADA, LISP,

DBASE or whatever.