A 68HC11 Subroutine For Error Control Coding Using The CCITT V.41 Polynomial Code

David Evans, 1999							Revision 4.

Table of Contents

1	Introduction	3

2	What the Software Computes	3

2.1	Encoding Data Frames for Transmission	4

2.2	Decoding Received Data Frames	4

2.3	CCITT V.41 Performance Characteristics	5

3	Data Variables and Symbols	5

4	Encoding Data Blocks for Transmission	8

5	Decoding Received Data Blocks	9

DISCLAIMER: While all efforts have been made to ensure the information contained within this document is accurate and correct, the author does not assume liability arising out of the application or use of any information or software described herein.  Please forward any suggestions or corrections to: arkaydia@hotmail.com

A 68HC11 Subroutine For Error Control Coding Using The CCITT V.41 Polynomial Code.

# Introduction

This document provides a description of error control coding software developed as a subroutine for use with the Motorola 68HC11 microcontroller.  The subroutine described uses a Cyclic Redundancy Code (CRC) for the purpose of data error detection and conforms to the CCITT recommendation V.41.  The name of the subroutine is crc_V41, and has been written in a way that allows it to be tailored to any 68HC11 system quickly and easily by changing symbol values at the beginning of the source code.

The following sections of this software guide provide all the necessary information to allow the reader to tailor the software to their specific system and encode and decode blocks of data.  A detailed description of software operation is not provided, however the crc_V41 source code is listed in appendix A for those who are interested.

# What the Software Computes

In this section, what crc_V41 computes is precisely defined.  Polynomial codes or Cyclic Redundancy Codes as they are commonly referred to, treat bit-strings as polynomials.  However, as binary digits may take on only one of two values, the coefficients of polynomials used in polynomial codes are either 1 or 0.

Consider a sequence of binary data that is k bits in length.  This bit-string may be considered to be the coefficients of a polynomial M(x) of degree k – 1, as shown below.

M(x) = xk – 1 + xk – 2 + xk – 3 + . . . + xk – k

As all coefficients will be either 1 or 0, the polynomial arithmetic is performed modulo 2.  Addition and subtraction operations using modulo 2 have no carries or borrows, respectively.  In fact, both of these operations are identical to Exclusive OR logic.  When polynomial codes are used as a form of error control, both communication end-points must choose a common generator polynomial G(x).  Let the generator polynomial be of degree j.

The crc_V41 subroutine uses the generator polynomial specified in the CCITT V.41 recommendation.  The generator polynomial used is G(x) = x16+x12+x5+1.  This polynomial is represented in the crc_V41 subroutine as a string of 17 bits:

G[n] = [1 0 0 0 1 0 0 0 0 0 0 1 0 0 0 0 1]

## 2.1	Encoding Data Frames for Transmission

To encode a block of data M(x) prior to transmission, j bits having a value of 0 are appended to the least significant end of M(x).  The data frame now contains k + j bits and corresponds to the polynomial x jM(x).

The polynomial x jM(x) is then divided by the generator polynomial G(x) using modulo 2 long division.  When a remainder is produced, it is always of length j or fewer bits, and is subtracted from the polynomial x jM(x) using modulo 2 subtraction as a final encoding step.  The resulting bit-string is the checksummed data frame ready for transmission and as a consequence it is called T(x).

## Decoding Received Data Frames

To decode a received block of data, the receiving end-point must use the same generator polynomial G(x) as the sender, as stated in the previous section.  The processes involved in decoding received data frames using polynomial codes are marginally simpler than those needed for encoding.

When the checksummed data frame T(x) is received, it is divided by the generator polynomial G(x).  If the remainder of this division is non-zero, the received data frame T(x) is assumed to be corrupted.

Let E(x) be a polynomial representing a transmission error.  In such a case, crc_V41 will compute [T(x) + E(x)]/G(x) = E(x)/G(x), since T(x)/G(x) will always equal zero.

## CCITT V.41 Performance Characteristics

Although the CCITT V.41 recommendation is extremely robust, as with all other error control strategies in existence, it is not 100% foolproof.  That is, a small chance exists that corrupted data will go unnoticed.  However, the chances of this event occurring are very remote, as outlined below.

A 16-bit checksum such as that used by CRC-CCITT has the following performance characteristics:

- All single and double bit errors are detected;

- All errors with an odd number of bits are detected;

- All burst errors 16 bits or less in length are detected;

- 99.997% of 17-bit burst errors are detected;

- 99.998% of all bursts errors of length 18 or longer are detected.

Although outdated by today’s standards, the CCITT V.41 recommendation still provides a very effective technique for detecting data errors.

# Data Variables and Symbols

A number of variables and symbols are used by crc_V41 to allow the software to be easily tailored to any 68HC11 system.  It is therefore necessary for the user to have a complete understanding of these variables and symbols.  The data variables used for this software all reside within the on-chip SRAM to exploit the faster access time of the direct addressing mode and enhance the software’s performance.  The addresses of the data variables should be checked to ensure they do not contend with other variable addresses within the host system.

All variables and symbols used by crc_V41 are listed and explained below:

CRC_DIR 	Specifications are made with this variable as to whether data frames will be encoded or decoded.  A value must be assigned to this variable before calling the crc_V41 subroutine.  Data frames are encoded and decoded by assigning $00 or $01 to this variable, respectively.  If this variable is assigned any number other than $00 or $01, the subroutine will immediately return with failure.

CRC_RET 	Calling software can perform simple error checking functions using this variable.  On successful execution of the crc_V41 subroutine a value of $00 will be assigned to this variable.  Otherwise a value of $FF will be assigned indicating that the subroutine has returned with failure.  Successful execution is achieved following the encoding of data frames and the appending of the 16-bit checksum, and also following the decoding of a data frame containing no errors.  The crc_V41 subroutine returns with failure if the CRC_DIR variable has not been assigned a valid value, or if a decoded data frame is found to be corrupted.

TEMPOR1 	This variable is used as a temporary data buffer that is needed during both the encoding and decoding of data frames.

XOR4RX1 		These three variables are used to hold the 17-bit result of

XOR4RX2 		exclusive OR operations that are performed during the

XOR4RX3 		decoding process.

XOR4TX1 		These three variables are used to hold the 17-bit result of

XOR4TX2 		exclusive OR operations that are performed during the

XOR4TX3 		encoding process.

TX_DIVIDEND1	These three variables hold a 24-bit dividend that is used

TX_DIVIDEND2	during the encoding process.  The dividend contains a

TX_DIVIDEND3	section of the data frame that is being encoded.

RX_DIVIDEND1	These three variables hold a 24-bit dividend that is used

RX_DIVIDEND2	during the decoding process.  The dividend contains a

RX_DIVIDEND3	section of the data frame that is being decoded.

IN_FOR_COUNT	During the data encoding and decoding processes, ‘for’ loops are used to exploit the iterative nature of these processes.  This variable is used as a counter for one of the ‘for’ loops used.

OUT_FOR_COUNT	Similarly, this variable is also used as a counter by one the ‘for’ loops within the subroutine.

GEN_POLY1	Encoding and decoding data using CRCs involves treating

GEN_POLY2 	data as polynomial coefficients and dividing this data by a

GEN_POLY3 	specific generator polynomial.  CCITT V.41 uses a generator polynomial G(x) = x16+x12+x5+1.  These three symbols represent this generator polynomial as a string of 17 bits.

TX_BUFF	This symbol specifies the starting address of a data block located in memory that is to be encoded prior to transmission.  If possible, this data block should be placed within Page0 of the 68HC11 memory map to enhance the execution time of the software.

RX_BUFF	The starting address of a data block located in memory that is to be decoded is specified with this symbol.  Similar to the previous symbol, it is recommended that data blocks be placed within Page0.  Decoded and encoded data blocks must be the same number of bytes in length.

FINAL_CHECK 	This variable is used to assist in the sequencing of the encoding and decoding processes.

OUTER_COUNT	During the division steps of the encoding and decoding operations, this symbol is used to determine the number of certain division steps that are required.  This number will vary depending on the length of the data blocks being encoded and decoded.  For a given data block length of N bytes, this symbol must be assigned a value of N-1.  For simplicity, this symbol should be assigned a value in decimal format.

CHKSUM1	These two symbols specify the location of the 16-bit

CHKSUM2	polynomial checksum that is appended to an encoded data block.  CHKSUM1 and CHKSUM1 correspond to the most-significant and least-significant bytes of the checksum, respectively.  For a given data block length of N bytes, CHKSUM1 must be assigned a value of N, and CHKSUM2 must be assigned a value of N+1.

# Encoding Data Blocks for Transmission

The encoding of data frames using the crc_V41 subroutine begins in the calling software.  Prior to calling crc_V41 to encode a data block, the data must be stored in memory with a starting address defined by the TX_BUFF symbol.  A value of $00 must then be assigned to the CRC_DIR variable to indicate that data is to be encoded when the subroutine is called.  All registers are pushed onto the stack at the start subroutine, and pulled from the stack just prior to completion.

At the beginning of the encoding process 16 bits, each of them being 0, are appended to the end of the data block where the polynomial checksum will be attached.  The entire data block is then treated as a polynomial and divided by the generator polynomial using modulo 2 long division.

The remainder from this long division process is then attached to the end of the data block where the 16 zero bits were appended.

Upon successfully encoding the data block the CRC_RET variable is assigned a value of $00 and the crc_V41 subroutine returns.  The calling software can check the value of the CRC_RET variable to perform a simple form of error checking following the subroutine execution.  However, this checking is more applicable to the decoding process.

# 5	Decoding Received Data Blocks

The decoding of data frames using the crc_V41 subroutine begins in the calling software.  Prior to calling crc_V41 to decode a data block, the received data must be placed in memory with a starting address defined by the RX_BUFF symbol.  A value of $01 must then be assigned to the CRC_DIR variable to indicate that data is to be decoded when the subroutine is called.  All registers are pushed onto the stack at the beginning of the subroutine, and pulled from the stack just prior to completion.

The decoding steps that follow are identical to the steps used to compute the remainder in the encoding process.  However, instead appending 16 zero bits to the end of the data block and attaching the remainder to the end of the data block as was done for encoding, the value of the remainder is checked.  A remainder of zero indicates that the received data frame contains no errors.  In such cases crc_V41 will then assign a value of $00 to the CRC_RET variable to indicate successful completion with no detected data corruption, and then return.

If however the remainder is checked and found to be non-zero, the received data frame will be corrupt.  The crc_V41 subroutine will then assign a value of $FF to the CRC_RET variable and return.  The calling software will then be able to determine that the received data is corrupted, and take further action.  For example, in some systems a request for re-transmission of the data block found to be corrupted may be made.

Appendix A

crc_V41 source code

The source code for the crc_V41 subroutine is listed below in this appendix.  Although the source code for the encoding and decoding operations contain segments of code that are repeated, no further subroutines are used within the crc_V41 subroutine.  This is a deliberate move to enhance the execution time of the software.  Due to the highly iterative nature of the encoding and decoding procedures, further subroutines within the code associated with these procedures would lead to gross inefficiencies in the execution of the software, because of the ‘overhead clock cycles’ required with the calling of a subroutine.

The following source code is located in the <CRC_V41.ASM> file and can be down-loaded from the web-site this document was obtained from.

**********************************************************************

*

* CRC_V41 -- Encodes and decodes data blocks using CCITT V.41 CRC.

* ====================================================================

*

* David Evans, September 1999.                 Version No:`LOST_COUNT'

*

* This software subroutine encodes and decodes data frames using a

* CRC that conforms to the CCITT V.41 recommendation. The software has

* been written in a manner that allows it to be easily and quickly

* tailored to your 68HC11 system.  A comprehensive guide to using this

* software can be found at the web-site this software was downloaded

* from.

*

* Prior to installing this subroutine, ensure that the <ORG $E000>

* command on line 59 is removed.

*

* This software has been rigorously tested.  However, although all

* efforts have been made to ensure that this software contains no bugs

* or errors, the author DOES NOT assume liability arising out of the

* application or use of any of the information presented herein.

*

* The author appreciates feedback -- Please feel free to forward any

* complaints, suggestions, corrections or comments to:

*

* arkaydia@hotmail.com

*

**********************************************************************

CRC_DIR         EQU     $07

CRC_RET         EQU     $08

TEMPOR1         EQU     $09

XOR4RX1         EQU     $20

XOR4RX2         EQU     $21

XOR4RX3         EQU     $22

XOR4TX1         EQU     $23

XOR4TX2         EQU     $24

XOR4TX3         EQU     $25

TX_DIVIDEND1    EQU     $33

TX_DIVIDEND2    EQU     $34

TX_DIVIDEND3    EQU     $35

RX_DIVIDEND1    EQU     $36

RX_DIVIDEND2    EQU     $37

RX_DIVIDEND3    EQU     $38

IN_FOR_COUNT    EQU     $29

OUT_FOR_COUNT   EQU     $31

GEN_POLY1       EQU     $88

GEN_POLY2       EQU     $10

GEN_POLY3       EQU     $80

FINAL_CHECK     EQU     $30

TX_BUFF         EQU     $50

RX_BUFF         EQU     $B0

OUTER_COUNT     EQU     31

CHKSUM1         EQU     32

CHKSUM2         EQU     33

ORG     $E000

crc_V41 PSHA                    ;

PSHB                    ;

PSHX                    ;

PSHY                    ;

LDAA    CRC_DIR         ; Fetch direction flag.

CMPA    #$01            ; If request is decode,

BEQ     CRC_DEC         ; jump to decoding algorithm.

LDAA    CRC_DIR         ; Fetch direction flag again.

CMPA    #$00            ; If request is encode,

BEQ     JMP_ENK         ; jump to encoding algorithm.

LDAA    #$FF            ;

STAA    CRC_RET         ; Otherwise return with failure.

PULY                    ;

PULX                    ;

PULB                    ;

PULA                    ;

RTS                     ;

JMP_ENK JMP     CRC_ENC         ; Jump to encoding algorithm.

***********************************************************************

***********************************************************************

*

* Received data frames are DECODED using in this subroutine using CCITT

* V.41 Cyclic Redundancy Code recommendation.

*

***********************************************************************

***********************************************************************

CRC_DEC LDX     #RX_BUFF        ; Load RX buffer pointer.

LDAA    0,X             ; Place 1st three bytes in DIVIDEND...

STAA    RX_DIVIDEND1    ;

STAA    XOR4RX1         ;

LDAA    $01,X           ;

STAA    RX_DIVIDEND2    ;

STAA    XOR4RX2         ;

LDAA    $02,X           ;

STAA    RX_DIVIDEND3    ;

ANDA    #$80            ; Remove all but the MSB.

STAA    XOR4RX3         ;

LDAA    #OUTER_COUNT    ; Initialise counter for the 'outer'

STAA    OUT_FOR_COUNT   ; for loop.

LDAA    #$08            ; Initialise counter for the 'inner'

STAA    IN_FOR_COUNT    ; for loop.

INX                     ;

INX                     ;

OUT_4   LDAA    #$00            ;

STAA    $FINAL_CHECK    ;

RX_DIV  LDAA    #GEN_POLY1      ; Load accA with highest byte of G(x).

CMPA    XOR4RX1         ; Compare with highest XRes byte,

BLO     A000            ; & if G(x)<Dn, then XOR with G(x).

LDAA    #GEN_POLY1      ;

CMPA    XOR4RX1         ; Checking if G(x) > D(x).

BHI     A005            ;

LDAA    #GEN_POLY2      ;

CMPA    XOR4RX2         ;

BLO     A000            ;

LDAA    #GEN_POLY2      ;

CMPA    XOR4RX2         ;

BHI     A005            ; Branch if G(x) < D(x).

LDAA    #GEN_POLY3      ;

CMPA    XOR4RX3         ;

BHI     A005            ;

BRA     A000            ; Branch if G(x) == D(x).

A005    JMP     XOR_0           ; Else, G(x)>Dn, so XOR with zero.

A000    JMP     XOR_GX          ;

NTRY2   LSL     RX_DIVIDEND1    ; NOW SHIFT THE DIVIDEND 1 BIT LEFT....

LDAA    RX_DIVIDEND2    ;

ANDA    #$80            ;

LSRA                    ; Most significant bit is now

LSRA                    ; shifted sown to least significant.

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

STAA    TEMPOR1         ; Store in memory so it can be OR'd.

LDAB    RX_DIVIDEND1    ;

ORAB    TEMPOR1         ;

STAB    RX_DIVIDEND1    ;

LSL     RX_DIVIDEND2    ;

LDAA    RX_DIVIDEND3    ;

ANDA    #$80            ;

LSRA                    ; Most significant bit is now

LSRA                    ; shifted sown to least significant.

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

STAA    TEMPOR1         ; Store in memory so it can be OR'd.

LDAB    RX_DIVIDEND2    ;

ORAB    TEMPOR1         ;

STAB    RX_DIVIDEND2    ;

LSL     RX_DIVIDEND3    ;

* Now shift the XOR result left 1 bit...

*---------------------------------------

LSL     XOR4RX1         ; Shift the first byte to 1 bit left.

LDAA    XOR4RX1         ; Put shifted byte in accA,

LDAB    XOR4RX2         ; & put the next byte in accB.

LSRB                    ;

LSRB                    ; The next several lines shift the

LSRB                    ; most significant bit down to the

LSRB                    ; least significant bit.

LSRB                    ;

LSRB                    ;

LSRB                    ;

STAB    TEMPOR1         ; Generic buffer.

ORAA    TEMPOR1         ; Concatenate LSB to accA.

STAA    XOR4RX1         ; Put new byte in 1st Xres space.

LSL     XOR4RX2         ; Shift 2nd XRes byte left 1 bit,

LDAA    XOR4RX2         ; & put it in accA.

LDAB    XOR4RX3         ; Put last XRes byte in accB,

LSRB                    ;

LSRB                    ; The next several lines shift the

LSRB                    ; most significant bit down to the

LSRB                    ; least significant bit.

LSRB                    ;

LSRB                    ;

LSRB                    ;

STAB    TEMPOR1         ;

ORAA    TEMPOR1         ; Concatenate LSB to accA.

STAA    XOR4RX2         ; Put the new byte in its place.

LSL     XOR4RX3         ;

* Now drag the next bit down for the next division...

*----------------------------------------------------

LDAB    RX_DIVIDEND3    ; Load accA with 3rd quotient byte

ANDB    #$80            ; & keep only the MSB...

STAB    XOR4RX3         ; Then store it away.

DEC     IN_FOR_COUNT    ; Executed inner loop 8 times?

BNE     JMP2RX          ; Keep looping if not.

DEC     OUT_FOR_COUNT   ;

BNE     JMP_OUT         ; Keep looping.

JMP     A004            ; Jump to the next step if finished.

JMP2RX  JMP     RX_DIV          ;

JMP_OUT INX                     ;

LDAA    #$08            ; Reinitialise counter of the inner

STAA    IN_FOR_COUNT    ; for loop.

LDAA    0,X             ; Fetch next byte from data frame.

STAA    RX_DIVIDEND3    ;

ANDA    #$80            ;

STAA    XOR4RX3         ;

JMP     OUT_4           ;

* G1(x) was < D1 so XOR data with G(x)...

*========================================

XOR_GX  LDAA    #GEN_POLY1      ;

EORA    XOR4RX1         ; XOR 1st XRes byte with 1st G(X) byte.

STAA    XOR4RX1         ; Store it in XRes buffer.

LDAA    #GEN_POLY2      ; Load accA with next G(x) byte,

EORA    XOR4RX2         ; & XOR it with the 2nd XRes byte.

STAA    XOR4RX2         ; Store result in XRes buffer.

LDAA    #GEN_POLY3      ; Load accA with last bit of G(x),

LDAB    XOR4RX3         ; Load accB with 3rd XRes byte,

STAB    TEMPOR1         ; & store in a generic buffer.

EORA    TEMPOR1         ; XOR 3rd data byte & last bit of G(x).

ANDA    #$80            ; Get rid of all but the MSB,

STAA    XOR4RX3         ; & store it away.

JMP     NTRY2           ; Begin the next long div. step...

* G1(x) was > D1 so XOR data with zero...

*========================================

XOR_0   LDAA    #$00            ;

EORA    XOR4RX1         ; XOR 1st XRes byte with zero.

STAA    XOR4RX1         ; Store it in XRes buffer.

LDAA    #$00            ; Clear accA,

EORA    XOR4RX2         ; & XOR it with the 2nd XRes byte.

STAA    XOR4RX2         ; Store result in XRes buffer.

LDAA    #$00            ; Clear accA.

LDAB    XOR4RX3         ; Load accB with 3rd XRes byte,

STAB    TEMPOR1         ; & store in a generic buffer.

EORA    TEMPOR1         ; XOR 3rd XRes byte & last bit of G(x).

ANDA    #$80            ; Get rid of all but the MSB,

STAA    XOR4RX3         ; & store it away.

JMP     NTRY2           ;

A004    INX                     ;

LDAA    0,X             ;

STAA    RX_DIVIDEND3    ;

ANDA    #$80            ;

STAA    XOR4RX3         ;

LDAA    #$07            ; Do this seven times...

STAA    IN_FOR_COUNT    ;

FINI    LDAA    #GEN_POLY1      ; Load accA with highest byte of G(x).

CMPA    XOR4RX1         ; Compare with highest XRes byte,

BLO     A007            ; & if G(x)<Dn, then XOR with G(x).

LDAA    #GEN_POLY1      ;

CMPA    XOR4RX1         ; Checking if G(x) > D(x).

BHI     A006            ;

LDAA    #GEN_POLY2      ;

CMPA    XOR4RX2         ;

BLO     A007            ;

LDAA    #GEN_POLY2      ;

CMPA    XOR4RX2         ;

BHI     A006            ; Branch if G(x) < D(x).

LDAA    #GEN_POLY3      ;

CMPA    XOR4RX3         ;

BHI     A006            ;

BRA     A007            ; Branch if G(x) == D(x).

A006    JMP     XOR_01          ; Else, G(x)>Dn, so XOR with zero.

A007    JMP     XOR_GX1

NTRY3   LSL     RX_DIVIDEND1    ; NOW SHIFT DIVIDEND 1 BIT LEFT....

LDAA    RX_DIVIDEND2    ;

ANDA    #$80            ;

LSRA                    ; Most significant bit is now

LSRA                    ; shifted sown to least significant.

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

STAA    TEMPOR1         ; Store in memory so it can be OR'd.

LDAB    RX_DIVIDEND1    ;

ORAB    TEMPOR1         ;

STAB    RX_DIVIDEND1    ;

LSL     RX_DIVIDEND2    ;

LDAA    RX_DIVIDEND3    ;

ANDA    #$80            ;

LSRA                    ; Most significant bit is now

LSRA                    ; shifted sown to least significant.

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

STAA    TEMPOR1         ; Store in memory so it can be OR'd.

LDAB    RX_DIVIDEND2    ;

ORAB    TEMPOR1         ;

STAB    RX_DIVIDEND2    ;

LSL     RX_DIVIDEND3    ;

* Now shift the XOR result left 1 bit...

*---------------------------------------

LSL     XOR4RX1         ; Shift the first byte to 1 bit left.

LDAA    XOR4RX1         ; Put shifted byte in accA,

LDAB    XOR4RX2         ; & put the next byte in accB.

LSRB                    ;

LSRB                    ; The next several lines shift the

LSRB                    ; most significant bit down to the

LSRB                    ; least significant bit.

LSRB                    ;

LSRB                    ;

LSRB                    ;

STAB    TEMPOR1         ; Generic buffer.

ORAA    TEMPOR1         ; Concatenate LSB to accA.

STAA    XOR4RX1         ; Put new byte in 1st Xres space.

LSL     XOR4RX2         ; Shift 2nd XRes byte left 1 bit,

LDAA    XOR4RX2         ; & put it in accA.

LDAB    XOR4RX3         ; Put last XRes byte in accB,

LSRB                    ;

LSRB                    ; The next several lines shift the

LSRB                    ; most significant bit down to the

LSRB                    ; least significant bit.

LSRB                    ;

LSRB                    ;

LSRB                    ;

STAB    TEMPOR1         ;

ORAA    TEMPOR1         ; Concatenate LSB to accA.

STAA    XOR4RX2         ; Put the new byte in its place.

LSL     XOR4RX3         ;

LDAB    RX_DIVIDEND3    ; Load accA with 3rd quotient byte

ANDB    #$80            ; & keep only the MSB...

STAB    XOR4RX3         ; Then store it away.

DEC     IN_FOR_COUNT    ; Executed loop 7 times?

BNE     JMP_FIN         ; Keep looping if not.

BRA     FINI_OR         ;

JMP_FIN JMP     FINI

* One final exclusive OR before finishing...

*-------------------------------------------

FINI_OR LDAA    #$55            ;

STAA    $FINAL_CHECK    ;

LDAA    #GEN_POLY1      ; Load accA with highest byte of G(x).

CMPA    XOR4RX1         ; Compare with highest XRes byte,

BLO     A008            ; & if G(x)<Dn, then XOR with G(x).

LDAA    #GEN_POLY1      ;

CMPA    XOR4RX1         ; Checking if G(x) > D(x).

BHI     A009            ;

LDAA    #GEN_POLY2      ;

CMPA    XOR4RX2         ;

BLO     A008            ;

LDAA    #GEN_POLY2      ;

CMPA    XOR4RX2         ;

BHI     A009            ; Branch if G(x) < D(x).

LDAA    #GEN_POLY3      ;

CMPA    XOR4RX3         ;

BHI     A009            ;

BRA     A008            ; Branch if G(x) == D(x).

A009    JMP     XOR_01          ; Else, G(x)>Dn, so XOR with zero.

A008    JMP     XOR_GX1

* Shift the final XOR result left 1 bit...

*---------------------------------------

NTRY4   LSL     XOR4RX1         ; Shift the first byte to 1 bit left.

LDAA    XOR4RX1         ; Put shifted byte in accA,

LDAB    XOR4RX2         ; & put the next byte in accB.

LSRB                    ;

LSRB                    ; The next several lines shift the

LSRB                    ; most significant bit down to the

LSRB                    ; least significant bit.

LSRB                    ;

LSRB                    ;

LSRB                    ;

STAB    TEMPOR1         ; Generic buffer.

ORAA    TEMPOR1         ; Concatenate LSB to accA.

STAA    XOR4RX1         ; Put new byte in 1st Xres space.

LSL     XOR4RX2         ; Shift 2nd XRes byte left 1 bit,

LDAA    XOR4RX2         ; & put it in accA.

LDAB    XOR4RX3         ; Put last XRes byte in accB,

LSRB                    ;

LSRB                    ; The next several lines shift the

LSRB                    ; most significant bit down to the

LSRB                    ; least significant bit.

LSRB                    ;

LSRB                    ;

LSRB                    ;

STAB    TEMPOR1         ;

ORAA    TEMPOR1         ; Concatenate LSB to accA.

STAA    XOR4RX2         ; Put the new byte in its place.

LSL     XOR4RX3         ;

LDAA    XOR4RX1         ; Check the remainder...

CMPA    #$00            ;

BNE     FAIL            ; Return with failure if it's not 0.

LDAA    XOR4RX2         ;

CMPA    #$00            ;

BNE     FAIL            ;

LDAA    #$00            ;

STAA    CRC_DIR         ;

PULY                    ;

PULX                    ;

PULB                    ;

PULA                    ;

RTS                     ; Otherwise return with success!

FAIL    LDAA    #$FF            ;

STAA    CRC_DIR         ;

PULY                    ;

PULX                    ;

PULB                    ;

PULA                    ;

RTS                     ;

* G1(x) was < D1 so XOR data with G(x)...

*========================================

XOR_GX1 LDAA    #GEN_POLY1      ;

EORA    XOR4RX1         ; XOR 1st XRes byte with 1st G(X) byte.

STAA    XOR4RX1         ; Store it in XRes buffer.

LDAA    #GEN_POLY2      ; Load accA with next G(x) byte,

EORA    XOR4RX2         ; & XOR it with the 2nd XRes byte.

STAA    XOR4RX2         ; Store result in XRes buffer.

LDAA    #GEN_POLY3      ; Load accA with last bit of G(x),

LDAB    XOR4RX3         ; Load accB with 3rd XRes byte,

STAB    TEMPOR1         ; & store in a generic buffer.

EORA    TEMPOR1         ; XOR 3rd data byte & last bit of G(x).

ANDA    #$80            ; Get rid of all but the MSB,

STAA    XOR4RX3         ; & store it away.

LDAA    $FINAL_CHECK    ;

CMPA    #$55            ;

BEQ     A002            ;

JMP     NTRY3           ; Begin the next long div. step...

A002    JMP     NTRY4           ;

* G1(x) was > D1 so XOR data with zero...

*========================================

XOR_01  LDAA    #$00            ; Clear accA.

EORA    XOR4RX1         ; XOR 1st XRes byte with zero.

STAA    XOR4RX1         ; Store it in XRes buffer.

LDAA    #$00            ; Clear accA,

EORA    XOR4RX2         ; & XOR it with the 2nd XRes byte.

STAA    XOR4RX2         ; Store result in XRes buffer.

LDAA    #$00            ; Clear accA.

LDAB    XOR4RX3         ; Load accB with 3rd XRes byte,

STAB    TEMPOR1         ; & store in a generic buffer.

EORA    TEMPOR1         ; XOR 3rd XRes byte & last bit of G(x).

ANDA    #$80            ; Get rid of all but the MSB,

STAA    XOR4RX3         ; & store it away.

LDAA    $FINAL_CHECK    ;

CMPA    #$55            ;

BEQ     A003            ;

JMP     NTRY3           ;

A003    JMP     NTRY4

*^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

***********************************************************************

***********************************************************************

*

* This routine ENCODES a block of data to be transmitted using the

* CCITT V41 Cyclic Redundancy Code.

*

***********************************************************************

***********************************************************************

*^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

CRC_ENC LDX     #TX_BUFF        ; Load Tx buffer pointer.

LDAA    #$00            ;

STAA    CHKSUM1,X       ; Append 16 0's to the checksum area.

STAA    CHKSUM2,X       ;

LDAA    0,X             ; Place 1st three bytes in DIVIDEND...

STAA    TX_DIVIDEND1    ;

STAA    XOR4TX1         ;

LDAA    $01,X           ;

STAA    TX_DIVIDEND2    ;

STAA    XOR4TX2         ;

LDAA    $02,X           ;

STAA    TX_DIVIDEND3    ;

ANDA    #$80            ; Remove all but the MSB.

STAA    XOR4TX3         ;

LDAA    #OUTER_COUNT    ; Initialise counter for the 'outer'

STAA    OUT_FOR_COUNT   ; for loop.

LDAA    #$08            ; Initialise counter for the 'inner'

STAA    IN_FOR_COUNT    ; for loop.

INX                     ;

INX                     ;

OUT_FOR LDAA    #$00            ;

STAA    $FINAL_CHECK    ;

TX_DIV  LDAA    #GEN_POLY1      ; Load accA with highest byte of G(x).

CMPA    XOR4TX1         ; Compare with highest XRes byte,

BLO     B000            ; & if G(x)<Dn, then XOR with G(x).

LDAA    #GEN_POLY1      ;

CMPA    XOR4TX1         ; Checking if G(x) > D(x).

BHI     B005            ;

LDAA    #GEN_POLY2      ;

CMPA    XOR4TX2         ;

BLO     B000            ;

LDAA    #GEN_POLY2      ;

CMPA    XOR4TX2         ;

BHI     B005            ; Branch if G(x) < D(x).

LDAA    #GEN_POLY3      ;

CMPA    XOR4TX3         ;

BHI     B005            ;

BRA     B000            ; Branch if G(x) == D(x).

B005    JMP     XOR0            ; Else, G(x)>Dn, so XOR with zero.

B000    JMP     XORGX           ;

ENTRY2  LSL     TX_DIVIDEND1    ; NOW SHIFT DIVIDEND 1 BIT LEFT....

LDAA    TX_DIVIDEND2    ;

ANDA    #$80            ;

LSRA                    ; Most significant bit is now

LSRA                    ; shifted sown to least significant.

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

STAA    TEMPOR1         ; Store in memory so it can be OR'd.

LDAB    TX_DIVIDEND1    ;

ORAB    TEMPOR1         ;

STAB    TX_DIVIDEND1    ;

LSL     TX_DIVIDEND2    ;

LDAA    TX_DIVIDEND3    ;

ANDA    #$80            ;

LSRA                    ; Most significant bit is now

LSRA                    ; shifted sown to least significant.

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

STAA    TEMPOR1         ; Store in memory so it can be OR'd.

LDAB    TX_DIVIDEND2    ;

ORAB    TEMPOR1         ;

STAB    TX_DIVIDEND2    ;

LSL     TX_DIVIDEND3    ;

* Now shift the XOR result left 1 bit...

*---------------------------------------

LSL     XOR4TX1         ; Shift the first byte to 1 bit left.

LDAA    XOR4TX1         ; Put shifted byte in accA,

LDAB    XOR4TX2         ; & put the next byte in accB.

LSRB                    ;

LSRB                    ; The next several lines shift the

LSRB                    ; most significant bit down to the

LSRB                    ; least significant bit.

LSRB                    ;

LSRB                    ;

LSRB                    ;

STAB    TEMPOR1         ; Generic buffer.

ORAA    TEMPOR1         ; Concatenate LSB to accA.

STAA    XOR4TX1         ; Put new byte in 1st Xres space.

LSL     XOR4TX2         ; Shift 2nd XRes byte left 1 bit,

LDAA    XOR4TX2         ; & put it in accA.

LDAB    XOR4TX3         ; Put last XRes byte in accB,

LSRB                    ;

LSRB                    ; The next several lines shift the

LSRB                    ; most significant bit down to the

LSRB                    ; least significant bit.

LSRB                    ;

LSRB                    ;

LSRB                    ;

STAB    TEMPOR1         ;

ORAA    TEMPOR1         ; Concatenate LSB to accA.

STAA    XOR4TX2         ; Put the new byte in its place.

LSL     XOR4TX3         ;

* Now drag the next bit down for the next division...

*----------------------------------------------------

LDAB    TX_DIVIDEND3    ; Load accA with 3rd quotient byte

ANDB    #$80            ; & keep only the MSB...

STAB    XOR4TX3         ; Then store it away.

DEC     IN_FOR_COUNT    ; Executed inner loop 8 times?

BNE     JMP2TX          ; Keep looping if not.

DEC     OUT_FOR_COUNT   ;

BNE     JMP2OUT         ; Keep looping otherwise.

JMP     B004            ; Jump to next step if finished.

JMP2TX  JMP     TX_DIV          ;

JMP2OUT INX                     ;

LDAA    #$08            ; Reinitialise counter of the inner

STAA    IN_FOR_COUNT    ; for loop.

LDAA    0,X             ; Fetch next byte from data frame.

STAA    TX_DIVIDEND3    ;

ANDA    #$80            ;

STAA    XOR4TX3         ;

JMP     OUT_FOR         ;

* G1(x) was < D1 so XOR data with G(x)...

*========================================

XORGX   LDAA    #GEN_POLY1      ;

EORA    XOR4TX1         ; XOR 1st XRes byte with 1st G(X) byte.

STAA    XOR4TX1         ; Store it in XRes buffer.

LDAA    #GEN_POLY2      ; Load accA with next G(x) byte,

EORA    XOR4TX2         ; & XOR it with the 2nd XRes byte.

STAA    XOR4TX2         ; Store result in XRes buffer.

LDAA    #GEN_POLY3      ; Load accA with last bit of G(x),

LDAB    XOR4TX3         ; Load accB with 3rd XRes byte,

STAB    TEMPOR1         ; & store in a generic buffer.

EORA    TEMPOR1         ; XOR 3rd data byte & last bit of G(x).

ANDA    #$80            ; Get rid of all but the MSB,

STAA    XOR4TX3         ; & store it away.

JMP     ENTRY2          ; Begin the next long div. step...

* G1(x) was > D1 so XOR data with zero...

*========================================

XOR0    LDAA    #$00            ;

EORA    XOR4TX1         ; XOR 1st XRes byte with zero.

STAA    XOR4TX1         ; Store it in XRes buffer.

LDAA    #$00            ; Clear accA,

EORA    XOR4TX2         ; & XOR it with the 2nd XRes byte.

STAA    XOR4TX2         ; Store result in XRes buffer.

LDAA    #$00            ; Clear accA.

LDAB    XOR4TX3         ; Load accB with 3rd XRes byte,

STAB    TEMPOR1         ; & store in a generic buffer.

EORA    TEMPOR1         ; XOR 3rd XRes byte & last bit of G(x).

ANDA    #$80            ; Get rid of all but the MSB,

STAA    XOR4TX3         ; & store it away.

JMP     ENTRY2          ;

B004    INX                     ;

LDAA    0,X             ;

STAA    TX_DIVIDEND3    ;

ANDA    #$80            ;

STAA    XOR4TX3         ;

LDAA    #$07            ; Do this seven times...

STAA    IN_FOR_COUNT    ;

FIN     LDAA    #GEN_POLY1      ; Load accA with highest byte of G(x).

CMPA    XOR4TX1         ; Compare with highest XRes byte,

BLO     B007            ; & if G(x)<Dn, then XOR with G(x).

LDAA    #GEN_POLY1      ;

CMPA    XOR4TX1         ; Checking if G(x) > D(x).

BHI     B006            ;

LDAA    #GEN_POLY2      ;

CMPA    XOR4TX2         ;

BLO     B007            ;

LDAA    #GEN_POLY2      ;

CMPA    XOR4TX2         ;

BHI     B006            ; Branch if G(x) < D(x).

LDAA    #GEN_POLY3      ;

CMPA    XOR4TX3         ;

BHI     B006            ;

BRA     B007            ; Branch if G(x) == D(x).

B006    JMP     XOR01           ; Else, G(x)>Dn, so XOR with zero.

B007    JMP     XORGX1          ;

ENTRY3  LSL     TX_DIVIDEND1    ; SHIFT THE DIVIDEND 1 BIT LEFT....

LDAA    TX_DIVIDEND2    ;

ANDA    #$80            ;

LSRA                    ; Most significant bit is now

LSRA                    ; shifted sown to least significant.

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

STAA    TEMPOR1         ; Store in memory so it can be OR'd.

LDAB    TX_DIVIDEND1    ;

ORAB    TEMPOR1         ;

STAB    TX_DIVIDEND1    ;

LSL     TX_DIVIDEND2    ;

LDAA    TX_DIVIDEND3    ;

ANDA    #$80            ;

LSRA                    ; Most significant bit is now

LSRA                    ; shifted sown to least significant.

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

LSRA                    ;

STAA    TEMPOR1         ; Store in memory so it can be OR'd.

LDAB    TX_DIVIDEND2    ;

ORAB    TEMPOR1         ;

STAB    TX_DIVIDEND2    ;

LSL     TX_DIVIDEND3    ;

* Now shift the XOR result left 1 bit...

*---------------------------------------

LSL     XOR4TX1         ; Shift the first byte to 1 bit left.

LDAA    XOR4TX1         ; Put shifted byte in accA,

LDAB    XOR4TX2         ; & put the next byte in accB.

LSRB                    ;

LSRB                    ; The next several lines shift the

LSRB                    ; most significant bit down to the

LSRB                    ; least significant bit.

LSRB                    ;

LSRB                    ;

LSRB                    ;

STAB    TEMPOR1         ; Generic buffer.

ORAA    TEMPOR1         ; Concatenate LSB to accA.

STAA    XOR4TX1         ; Put new byte in 1st Xres space.

LSL     XOR4TX2         ; Shift 2nd XRes byte left 1 bit,

LDAA    XOR4TX2         ; & put it in accA.

LDAB    XOR4TX3         ; Put last XRes byte in accB,

LSRB                    ;

LSRB                    ; The next several lines shift the

LSRB                    ; most significant bit down to the

LSRB                    ; least significant bit.

LSRB                    ;

LSRB                    ;

LSRB                    ;

STAB    TEMPOR1         ;

ORAA    TEMPOR1         ; Concatenate LSB to accA.

STAA    XOR4TX2         ; Put the new byte in its place.

LSL     XOR4TX3         ;

LDAB    TX_DIVIDEND3    ; Load accA with 3rd quotient byte

ANDB    #$80            ; & keep only the MSB...

STAB    XOR4TX3         ; Then store it away.

DEC     IN_FOR_COUNT    ; Executed loop 7 times?

BNE     JMP2FIN         ; Keep looping if not.

BRA     FIN_XOR         ;

JMP2FIN JMP     FIN

* One final exclusive OR before finishing...

*-------------------------------------------

FIN_XOR LDAA    #$55            ;

STAA    $FINAL_CHECK    ;

LDAA    #GEN_POLY1      ; Load accA with highest byte of G(x).

CMPA    XOR4TX1         ; Compare with highest XRes byte,

BLO     B008            ; & if G(x)<Dn, then XOR with G(x).

LDAA    #GEN_POLY1      ;

CMPA    XOR4TX1         ; Checking if G(x) > D(x).

BHI     B009            ;

LDAA    #GEN_POLY2      ;

CMPA    XOR4TX2         ;

BLO     B008            ;

LDAA    #GEN_POLY2      ;

CMPA    XOR4TX2         ;

BHI     B009            ; Branch if G(x) < D(x).

LDAA    #GEN_POLY3      ;

CMPA    XOR4TX3         ;

BHI     B009            ;

BRA     B008            ; Branch if G(x) == D(x).

B009    JMP     XOR01           ; Else, G(x)>Dn, so XOR with zero.

B008    JMP     XORGX1

* Shift the final XOR result left 1 bit...

*---------------------------------------

ENTRY4  LSL     XOR4TX1         ; Shift the first byte to 1 bit left.

LDAA    XOR4TX1         ; Put shifted byte in accA,

LDAB    XOR4TX2         ; & put the next byte in accB.

LSRB                    ;

LSRB                    ; The next several lines shift the

LSRB                    ; most significant bit down to the

LSRB                    ; least significant bit.

LSRB                    ;

LSRB                    ;

LSRB                    ;

STAB    TEMPOR1         ; Generic buffer.

ORAA    TEMPOR1         ; Concatenate LSB to accA.

STAA    XOR4TX1         ; Put new byte in 1st Xres space.

LSL     XOR4TX2         ; Shift 2nd XRes byte left 1 bit,

LDAA    XOR4TX2         ; & put it in accA.

LDAB    XOR4TX3         ; Put last XRes byte in accB,

LSRB                    ;

LSRB                    ; The next several lines shift the

LSRB                    ; most significant bit down to the

LSRB                    ; least significant bit.

LSRB                    ;

LSRB                    ;

LSRB                    ;

STAB    TEMPOR1         ;

ORAA    TEMPOR1         ; Concatenate LSB to accA.

STAA    XOR4TX2         ; Put the new byte in its place.

LSL     XOR4TX3         ;

LDX     #TX_BUFF        ; Appending calculated checksum to

LDAA    XOR4TX1         ; the data frame to be transmitted.

STAA    CHKSUM1,X       ;

LDAA    XOR4TX2         ;

STAA    CHKSUM2,X       ;

LDAA    #$00            ;

STAA    CRC_DIR         ;

PULY                    ;

PULX                    ;

PULB                    ;

PULA                    ;

RTS                     ; Return from subroutine with success.

* G1(x) was < D1 so XOR data with G(x)...

*========================================

XORGX1  LDAA    #GEN_POLY1      ;

EORA    XOR4TX1         ; XOR 1st XRes byte with 1st G(X) byte.

STAA    XOR4TX1         ; Store it in XRes buffer.

LDAA    #GEN_POLY2      ; Load accA with next G(x) byte,

EORA    XOR4TX2         ; & XOR it with the 2nd XRes byte.

STAA    XOR4TX2         ; Store result in XRes buffer.

LDAA    #GEN_POLY3      ; Load accA with last bit of G(x),

LDAB    XOR4TX3         ; Load accB with 3rd XRes byte,

STAB    TEMPOR1         ; & store in a generic buffer.

EORA    TEMPOR1         ; XOR 3rd data byte & last bit of G(x).

ANDA    #$80            ; Get rid of all but the MSB,

STAA    XOR4TX3         ; & store it away.

LDAA    $FINAL_CHECK    ;

CMPA    #$55            ;

BEQ     B002            ;

JMP     ENTRY3          ; Begin the next long div. step...

B002    JMP     ENTRY4

* G1(x) was > D1 so XOR data with zero...

*========================================

XOR01   LDAA    #$00            ; Clear accA.

EORA    XOR4TX1         ; XOR 1st XRes byte with zero.

STAA    XOR4TX1         ; Store it in XRes buffer.

LDAA    #$00            ; Clear accA,

EORA    XOR4TX2         ; & XOR it with the 2nd XRes byte.

STAA    XOR4TX2         ; Store result in XRes buffer.

LDAA    #$00            ; Clear accA.

LDAB    XOR4TX3         ; Load accB with 3rd XRes byte,

STAB    TEMPOR1         ; & store in a generic buffer.

EORA    TEMPOR1         ; XOR 3rd XRes byte & last bit of G(x).

ANDA    #$80            ; Get rid of all but the MSB,

STAA    XOR4TX3         ; & store it away.

LDAA    $FINAL_CHECK    ;

CMPA    #$55            ;

BEQ     B003            ;

JMP     ENTRY3          ;

B003    JMP     ENTRY4          ;

* Version No.`LOST_COUNT', late Saturday afternoon, September 18, 1999.

*

* Please forward any comments, suggestions, or corrections to:

* arkaydia@hotmail.com