RTGroup5ConnRev0

| Part# | Location | Vlaue | Description |
| --- | --- | --- | --- |
| 16300008 | C1 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 16300008 | C11 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 16300008 | C12 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 16300008 | C15 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| XXXXXXXX | C16 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 16300008 | C19 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 16300008 | C2 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 16300008 | C20 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 16300008 | C23 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| XXXXXXXX | C24 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 16300008 | C27 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 16300008 | C28 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 16300008 | C31 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| XXXXXXXX | C32 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 12000080 | C37 | 2200U/35/ELRM | 2200U 35V RL ALUM ELECT- .3LS |
| 12000080 | C38 | 2200U/35/ELRM | 2200U 35V RL ALUM ELECT- .3LS |
| 12000080 | C39 | 2200U/35/ELRM | 2200U 35V RL ALUM ELECT- .3LS |
| 12000045 | C40 | 220U/63/ELRM | 220U 63V RL ALUM ELECT- .2 LS |
| 16300008 | C5 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| XXXXXXXX | C6 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 70000041 | D1 | FR604 | RECTIFIER 6A 400PIV FASTRECOVERY |
| 70000041 | D2 | FR604 | RECTIFIER 6A 400PIV FASTRECOVERY |
| 70000041 | D3 | FR604 | RECTIFIER 6A 400PIV FASTRECOVERY |
| 27000006 | D4 | FR204 | DIODE (2A) |
| 24000051 | HDR1 | HDR156-3 | .156 CENTER 3 WAY ST LOCKINGHEADER |
| 82300076 | HDR10 | RIB&SKT-5X2 | 5X2 DUAL ROW TRANSHDR+RIB+SKT |
| 23000125 | HDR11 | BOXHDR-10RA | 5X2 RA BOX HEADER W/ GOLDPINS |
| 24300025 | HDR12 | BOXSTOF-13X2 | 13X2 DUAL ROW BOX MALE HDR-1.1" TALL |
| 24300018 | HDR6 | SKT7X2-CS | 7X2 DUAL ROW COMP SIDESOCKET-GOLD |
| 24300018 | HDR7 | SKT7X2-CS | 7X2 DUAL ROW COMP SIDESOCKET-GOLD |
| 24300018 | HDR8 | SKT7X2-CS | 7X2 DUAL ROW COMP SIDESOCKET-GOLD |
| 24300018 | HDR9 | SKT7X2-CS | 7X2 DUAL ROW COMP SIDESOCKET-GOLD |
| 23000119 | J2 | HIROSE-P | 7 PIN PANEL MNT CONN- PLUG |
| 26000042 | J21 | TRS-11J1393 | PERP PC MOUNT TRS JACKW/METAL BUSH 24mm HI |
| 26000042 | J22 | TRS-11J1393 | PERP PC MOUNT TRS JACKW/METAL BUSH 24mm HI |
| 26000042 | J23 | TRS-11J1393 | PERP PC MOUNT TRS JACKW/METAL BUSH 24mm HI |
| 26000030 | J24 | X901-03 | VERT PC MOUNT 3 PIN MALE XLR |
| 26000042 | J25 | TRS-11J1393 | PERP PC MOUNT TRS JACKW/METAL BUSH 24mm HI |
| 26000042 | J26 | TRS-11J1393 | PERP PC MOUNT TRS JACKW/METAL BUSH 24mm HI |
| 26000042 | J27 | TRS-11J1393 | PERP PC MOUNT TRS JACKW/METAL BUSH 24mm HI |
| 26000030 | J28 | X901-03 | VERT PC MOUNT 3 PIN MALE XLR |
| 26000042 | J29 | TRS-11J1393 | PERP PC MOUNT TRS JACKW/METAL BUSH 24mm HI |
| 26000042 | J30 | TRS-11J1393 | PERP PC MOUNT TRS JACKW/METAL BUSH 24mm HI |
| 26000042 | J31 | TRS-11J1393 | PERP PC MOUNT TRS JACKW/METAL BUSH 24mm HI |
| 26000030 | J32 | X901-03 | VERT PC MOUNT 3 PIN MALE XLR |
| 26000042 | J33 | TRS-11J1393 | PERP PC MOUNT TRS JACKW/METAL BUSH 24mm HI |
| 26000042 | J34 | TRS-11J1393 | PERP PC MOUNT TRS JACKW/METAL BUSH 24mm HI |
| 26000042 | J35 | TRS-11J1393 | PERP PC MOUNT TRS JACKW/METAL BUSH 24mm HI |
| 26000030 | J36 | X901-03 | VERT PC MOUNT 3 PIN MALE XLR |
| 90000003 | LUG1 | LUG2 | PC MOUNT .250 STRAIGHT LUG |
| 69700090 | PCB1 | PCB | X8 GROUP RT 5-8 CONNECTOR |
| 24000056 | PWR1 | HDR156-7 | .156 CENTER 7 WAY ST LOCKINGHEADER |
| 24000056 | PWR2 | HDR156-7 | .156 CENTER 7 WAY ST LOCKINGHEADER |
| 72600002 | R1 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R14 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R2 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R3 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R4 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 89100042 | SW1 | SW-SPEC12 | 2PDT LATCHING SW- VERT PCB |