XR20-24OutputConnRev0

| Part# | Location | Value | Description |
| --- | --- | --- | --- |
| EN-98500 | 4 FW1 | XR-WIRE | XR-20 MASTER FADER WIRE ASSY38 |
| EN-98500 | 4 FW2 | XR-WIRE | XR-20 MASTER FADER WIRE ASSY38 |
| EN-98500 | 4 FW3 | XR-WIRE | XR-20 MASTER FADER WIRE ASSY38 |
| EN-98500 | 4 RIB1 | PCB | X-RACK FADER WIRING29 |
| EN-98500 | 4 RIB2 | PCB | X-RACK FADER WIRING29 |
| 47300090 | BRKT1 | KEY8190 | PC MOUNT BRACKET #6-32 |
| 47300090 | BRKT2 | KEY8190 | PC MOUNT BRACKET #6-32 |
| 12000029 | C1 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C10 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300007 | C102 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C104 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C105 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300007 | C106 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C107 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C108 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C109 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300008 | C11 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 16300007 | C110 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C111 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C112 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C113 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300007 | C114 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C115 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C116 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C117 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300007 | C118 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C119 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300008 | C12 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 12000029 | C120 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C121 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300007 | C122 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C123 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C124 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C125 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300007 | C126 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C127 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C128 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C129 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C13 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300003 | C130 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C131 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C134 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C135 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C136 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C137 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C138 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C139 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300007 | C14 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300003 | C144 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C145 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C148 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C149 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300007 | C15 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300003 | C156 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C157 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C158 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C159 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300008 | C16 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 16300003 | C162 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C163 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C164 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C165 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C168 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C169 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300008 | C17 | 100P/100/CNAJ | 100PF 100V COG CER CAP AXIAL5% |
| 16300003 | C172 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C173 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C176 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C177 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 12000029 | C18 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C19 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C2 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C20 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300007 | C21 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300007 | C22 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300003 | C23 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C24 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C25 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300007 | C26 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300007 | C27 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000016 | C28 | 100U/25/ELRM | CAP RADIAL ELECT 100UF/25V .1LS |
| 16300003 | C29 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300007 | C3 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C30 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C31 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C32 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C33 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C34 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C35 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C36 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C37 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C38 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C39 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C4 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000016 | C40 | 100U/25/ELRM | CAP RADIAL ELECT 100UF/25V .1LS |
| 12000029 | C41 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300039 | C42 | U1/50/CXAK | 0.1/50V 10% AXIAL CER CAP-X7R |
| 12000016 | C43 | 100U/25/ELRM | CAP RADIAL ELECT 100UF/25V .1LS |
| 12000016 | C44 | 100U/25/ELRM | CAP RADIAL ELECT 100UF/25V .1LS |
| 12000016 | C45 | 100U/25/ELRM | CAP RADIAL ELECT 100UF/25V .1LS |
| 12000016 | C46 | 100U/25/ELRM | CAP RADIAL ELECT 100UF/25V .1LS |
| 12000016 | C47 | 100U/25/ELRM | CAP RADIAL ELECT 100UF/25V .1LS |
| 12000016 | C48 | 100U/25/ELRM | CAP RADIAL ELECT 100UF/25V .1LS |
| 12000029 | C49 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C5 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C50 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C51 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C52 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C53 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300003 | C54 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300003 | C55 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300007 | C56 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C57 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C58 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C59 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300007 | C6 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C60 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C61 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C62 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000016 | C63 | 100U/25/ELRM | CAP RADIAL ELECT 100UF/25V .1LS |
| 12000029 | C64 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C65 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 12000029 | C66 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300003 | C7 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300007 | C72 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300003 | C8 | U1/50/CZAM | 0.1/50V 20% AXIAL CER CAP-Z5U |
| 16300007 | C80 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C82 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C83 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300007 | C9 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 16300007 | C91 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 12000029 | C93 | 47U/25/ELRM | CAP ELECT RADIAL 47uF/25V .1LS |
| 16300007 | C94 | 22P/50/CNAJ | 22PF 50V COG AXIAL CER CAP 5% |
| 27000009 | D1 | 1N4148 | DIODE (500MW) .400LS |
| 82300148 | HDR1 | BOXHDR-34-CON | 34 PIN RIBBON SOCKET TOTRANSISTION ASSY |
| 24000056 | HDR10 | HDR156-7 | .156 CENTER 7 WAY ST LOCKINGHEADER |
| 24000574 | HDR11 | HDR100-8X1-.5 | 8 WAY SING ROW BOARD STACKER.500" |
| 24000574 | HDR12 | MHDRG-4 | 1/2 OF 8 WAY SING ROW 0.5"TALL MALE HEADER-GOLD |
| 24000574 | HDR13 | HDR100-8X1-.5 | 8 WAY SING ROW BOARD STACKER.500" |
| 24000574 | HDR14 | HDR100-8X1-.5 | 8 WAY SING ROW BOARD STACKER.500" |
| 24000573 | HDR15 | SKT8X1-CS | 8 WAY SING ROW FEMALERECEPT-BTM ENTRY |
| 24000569 | HDR16 | SKT4X1-CS | 4 WAY SING ROW FEMALERECEPT-BTM ENTRY |
| 24000573 | HDR17 | SKT8X1-CS | 8 WAY SING ROW FEMALERECEPT-BTM ENTRY |
| 24000573 | HDR18 | SKT8X1-CS | 8 WAY SING ROW FEMALERECEPT-BTM ENTRY |
| 24000573 | HDR19 | SKT8X1-CS | 8 WAY SING ROW FEMALERECEPT-BTM ENTRY |
| 82300148 | HDR2 | BOXHDR-34-CON | 34 PIN RIBBON SOCKET TOTRANSISTION ASSY |
| 24000573 | HDR20 | SKT8X1-CS | 8 WAY SING ROW FEMALERECEPT-BTM ENTRY |
| 24000573 | HDR21 | SKT8X1-CS | 8 WAY SING ROW FEMALERECEPT-BTM ENTRY |
| 82300076 | HDR3 | SKT&SKT-5X2 | 5X2 DUAL ROW TRANS HDR+RIB+TRANS HDR |
| 23000033 | HDR5 | SKT&SKT-5X2 | 2 X 5 BOX HEADER |
| 24000574 | HDR7 | HDR100-8X1-.5 | 8 WAY SING ROW BOARD STACKER.500" |
| 24000574 | HDR8 | HDR100-8X1-.5 | 8 WAY SING ROW BOARD STACKER .500" |
| 24000574 | HDR9 | HDR100-8X1-.5 | 8 WAY SING ROW BOARD STACKER.500" |
| 26000030 | J1 | X901-03 | VERT PC MOUNT 3 PIN MALE XLR |
| 26000032 | J10 | RCA-DUAL | DUAL RCA JACKS-GOLD PC MOUNT24MM HEIGHT |
| 26000042 | J11 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000042 | J12 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000032 | J13 | RCA-DUAL | DUAL RCA JACKS-GOLD PC MOUNT24MM HEIGHT |
| 26000042 | J14 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000042 | J15 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000042 | J16 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000042 | J17 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000042 | J18 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000042 | J19 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000030 | J2 | X901-03 | VERT PC MOUNT 3 PIN MALE XLR |
| 26000046 | J20 | TRS-N114BPCS | PERP PC MOUNT TRS JACK W/INSUL BUSH 24mm HI |
| 26000046 | J21 | TRS-N114BPCS | PERP PC MOUNT TRS JACK W/INSUL BUSH 24mm HI |
| 26000046 | J22 | TRS-N114BPCS | PERP PC MOUNT TRS JACK W/INSUL BUSH 24mm HI |
| 26000046 | J23 | TRS-N114BPCS | PERP PC MOUNT TRS JACK W/INSUL BUSH 24mm HI |
| 26000030 | J24 | X901-03 | VERT PC MOUNT 3 PIN MALE XLR |
| 26000030 | J25 | X901-03 | VERT PC MOUNT 3 PIN MALE XLR |
| 26000030 | J26 | X901-03 | VERT PC MOUNT 3 PIN MALE XLR |
| 26000030 | J27 | X901-03 | VERT PC MOUNT 3 PIN MALE XLR |
| 26000030 | J28 | X901-03 | VERT PC MOUNT 3 PIN MALE XLR |
| 26000030 | J29 | X901-03 | VERT PC MOUNT 3 PIN MALE XLR |
| 26000030 | J3 | X901-03 | VERT PC MOUNT 3 PIN MALE XLR |
| 26000042 | J30 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000042 | J31 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000042 | J32 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000042 | J33 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000042 | J34 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000042 | J35 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000046 | J36 | TRS-N114BPCS | PERP PC MOUNT TRS JACK W/INSUL BUSH 24mm HI |
| 26000046 | J37 | TRS-N114BPCS | PERP PC MOUNT TRS JACK W/INSUL BUSH 24mm HI |
| 26000046 | J38 | TRS-N114BPCS | PERP PC MOUNT TRS JACK W/INSUL BUSH 24mm HI |
| 26000046 | J39 | TRS-N114BPCS | PERP PC MOUNT TRS JACK W/INSUL BUSH 24mm HI |
| 26000046 | J4 | TRS-N114BPCS | PERP PC MOUNT TRS JACK W/INSUL BUSH 24mm HI |
| 26000046 | J40 | TRS-N114BPCS | PERP PC MOUNT TRS JACK W/INSUL BUSH 24mm HI |
| 26000046 | J41 | TRS-N114BPCS | PERP PC MOUNT TRS JACK W/INSUL BUSH 24mm HI |
| 26000046 | J5 | TRS-N114BPCS | PERP PC MOUNT TRS JACK W/INSUL BUSH 24mm HI |
| 26000046 | J6 | TRS-N114BPCS | PERP PC MOUNT TRS JACK W/INSUL BUSH 24mm HI |
| 26000032 | J7 | RCA-DUAL | DUAL RCA JACKS-GOLD PC MOUNT 24MM HEIGHT |
| 26000042 | J8 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| 26000042 | J9 | TRS-11J1393 | PERP PC MOUNT TRS JACK W/METAL BUSH 24mm HI |
| XXXXXXXX | MH1 | STOFF/440/500 | STANDOFF AL 3/16 HEX 4-40 X1/2 |
| XXXXXXXX | MH2 | STOFF/440/500 | STANDOFF AL 3/16 HEX 4-40 X1/2 |
| XXXXXXXX | MH3 | STOFF/440/500 | STANDOFF AL 3/16 HEX 4-40 X1/2 |
| XXXXXXXX | MH4 | STOFF/440/500 | STANDOFF AL 3/16 HEX 4-40 X1/2 |
| XXXXXXXX | MH5 | STOFF/440/500 | STANDOFF AL 3/16 HEX 4-40 X1/2 |
| 23000065 | MIDI1 | AMP5PINDIN | PCB RA SHIELDED 5 PIN DIN |
| 23000065 | MIDI2 | AMP5PINDIN | PCB RA SHIELDED 5 PIN DIN |
| 69700105 | PCB1 | PCB | X-RACK OUTPUT CONNECTOR PCB |
| 94000038 | Q1 | J174 | P CHAN FET TO-92 |
| 94000038 | Q2 | J174 | P CHAN FET TO-92 |
| 94000003 | Q3 | 2N5551 | NPN SWITCH TO-92 |
| 94000003 | Q4 | 2N5551 | NPN SWITCH TO-92 |
| 73500047 | R1 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R10 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R100 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R101 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R102 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R103 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R104 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R105 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R106 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R107 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R108 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500050 | R109 | 200K0 | 1/4W 1% MF RES- .400 LS |
| 73500100 | R11 | 1K50 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R110 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500039 | R111 | 20K0 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R112 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R113 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R114 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500035 | R115 | 8K25 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R116 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R117 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R118 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R119 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500100 | R12 | 1K50 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R120 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R121 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R122 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R123 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R124 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R13 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500029 | R131 | 3K01 | 1/4W 1% MF RES- .400 LS |
| 73500029 | R132 | 3K01 | 1/4W 1% MF RES- .400 LS |
| 73500011 | R133 | 221R | 1/4W 1% MF RES- .400 LS |
| 73500006 | R134 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R135 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R136 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R137 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R138 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R139 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R14 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R140 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R141 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R142 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R143 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R144 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R145 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R146 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R147 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R148 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R149 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500231 | R15 | 33K2 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R150 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500029 | R151 | 3K01 | 1/4W 1% MF RES- .400 LS |
| 73500023 | R152 | 1K0 | 1/4W 1% MF RES- .400 LS |
| 73500029 | R153 | 3K01 | 1/4W 1% MF RES- .400 LS |
| 73500023 | R154 | 1K0 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R155 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R156 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R157 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500127 | R158 | 6K81 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R159 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R16 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R160 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R161 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R162 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R163 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R164 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R165 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500127 | R166 | 6K81 | 1/4W 1% MF RES- .400 LS |
| 73500127 | R167 | 6K81 | 1/4W 1% MF RES- .400 LS |
| 73500127 | R168 | 6K81 | 1/4W 1% MF RES- .400 LS |
| 73500127 | R169 | 6K81 | 1/4W 1% MF RES- .400 LS |
| 73500035 | R17 | 8K25 | 1/4W 1% MF RES- .400 LS |
| 73500127 | R170 | 6K81 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R171 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R172 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R173 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R175 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R176 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R177 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R178 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R179 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500035 | R18 | 8K25 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R180 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R181 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R187 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R188 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R189 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R19 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R191 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R192 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R193 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R194 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R195 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R196 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R197 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R198 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R199 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500035 | R2 | 8K25 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R20 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R200 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R201 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R202 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R203 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R204 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R205 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R206 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R207 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R208 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R209 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500100 | R21 | 1K50 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R210 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R211 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R212 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R213 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R214 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R215 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R216 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R217 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R218 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R219 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500100 | R22 | 1K50 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R220 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R221 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R222 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R223 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R224 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R225 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R226 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R227 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R228 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R229 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500231 | R23 | 33K2 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R230 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R231 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R232 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R233 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R234 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R235 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R236 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R237 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R238 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R239 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500047 | R24 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R240 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R241 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R242 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R243 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R244 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R245 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R246 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R247 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R248 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R249 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R25 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R250 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R251 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R252 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R253 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R254 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R255 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R256 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500044 | R257 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R258 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R259 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R26 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R260 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R261 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R262 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R263 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R264 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R265 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R268 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R269 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R27 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R270 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R271 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R272 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R273 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R278 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R279 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R28 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R282 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R283 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R29 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R290 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R291 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R292 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R293 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R296 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R297 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R298 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R299 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500119 | R3 | 15K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R30 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R302 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R303 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R306 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R307 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500029 | R31 | 3K01 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R310 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R311 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500023 | R32 | 1K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R33 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R34 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R35 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R36 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R37 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R38 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500029 | R39 | 3K01 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R4 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500023 | R40 | 1K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R41 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R42 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R43 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R44 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R45 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R46 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R47 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R48 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R49 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500119 | R5 | 15K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R50 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R51 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R52 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500029 | R53 | 3K01 | 1/4W 1% MF RES- .400 LS |
| 73500029 | R54 | 3K01 | 1/4W 1% MF RES- .400 LS |
| 73500011 | R55 | 221R | 1/4W 1% MF RES- .400 LS |
| 73500006 | R56 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R57 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R58 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R59 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500035 | R6 | 8K25 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R60 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R61 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R62 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R63 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600001 | R64 | 10R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600001 | R65 | 10R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600001 | R66 | 10R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R67 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R68 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600001 | R69 | 10R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500035 | R7 | 8K25 | 1/4W 1% MF RES- .400 LS |
| 72600001 | R70 | 10R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500029 | R71 | 3K01 | 1/4W 1% MF RES- .400 LS |
| 73500029 | R72 | 3K01 | 1/4W 1% MF RES- .400 LS |
| 73500011 | R73 | 221R | 1/4W 1% MF RES- .400 LS |
| 73500044 | R74 | 47K5 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R75 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R76 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R77 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R78 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R79 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500035 | R8 | 8K25 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R80 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R81 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R82 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R83 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R84 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R85 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 72600002 | R86 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 72600002 | R87 | 22R-FP | 1/4W 5% FLAME PROOF- .400 LS |
| 73500036 | R88 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R89 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R9 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R90 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500047 | R91 | 100K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R92 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R93 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R94 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R95 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R96 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500006 | R97 | 100R0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R98 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 73500036 | R99 | 10K0 | 1/4W 1% MF RES- .400 LS |
| 39000015 | SC1 | SCREW/4-40BH | #4-40 X 1/4 SOCKET BUTTON HEAD SCREW SS BLACK |
| 39000015 | SC10 | SCREW/4-40BH | #4-40 X 1/4 SOCKET BUTTON HEAD SCREW SS BLACK |
| 39000015 | SC2 | SCREW/4-40BH | #4-40 X 1/4 SOCKET BUTTON HEAD SCREW SS BLACK |
| 39000015 | SC3 | SCREW/4-40BH | #4-40 X 1/4 SOCKET BUTTON HEAD SCREW SS BLACK |
| 39000015 | SC4 | SCREW/4-40BH | #4-40 X 1/4 SOCKET BUTTON HEAD SCREW SS BLACK |
| 39000015 | SC5 | SCREW/4-40BH | #4-40 X 1/4 SOCKET BUTTON HEAD SCREW SS BLACK |
| 39000015 | SC6 | SCREW/4-40BH | #4-40 X 1/4 SOCKET BUTTON HEAD SCREW SS BLACK |
| 39000015 | SC7 | SCREW/4-40BH | #4-40 X 1/4 SOCKET BUTTON HEAD SCREW SS BLACK |
| 39000015 | SC8 | SCREW/4-40BH | #4-40 X 1/4 SOCKET BUTTON HEAD SCREW SS BLACK |
| 39000015 | SC9 | SCREW/4-40BH | #4-40 X 1/4 SOCKET BUTTON HEAD SCREW SS BLACK |
| 85000003 | SKT22 | SOCKET8 | 8 PIN DIP SOCKET |
| 85000003 | SKT23 | SOCKET8 | 8 PIN DIP SOCKET |
| 85000003 | SKT24 | SOCKET8 | 8 PIN DIP SOCKET |
| 87600002 | ST1 | STOFF/440/500 | STANDOFF AL 3/16 HEX 4-40 X1/2 |
| 87600002 | ST2 | STOFF/440/500 | STANDOFF AL 3/16 HEX 4-40 X1/2 |
| 87600002 | ST3 | STOFF/440/500 | STANDOFF AL 3/16 HEX 4-40 X1/2 |
| 87600002 | ST4 | STOFF/440/500 | STANDOFF AL 3/16 HEX 4-40 X1/2 |
| 87600002 | ST5 | STOFF/440/500 | STANDOFF AL 3/16 HEX 4-40 X1/2 |
| 89100042 | SW1 | SW-SPEC12 | 2PDT LATCHING SW- VERT PCB |
| 89100042 | SW2 | SW-SPEC12 | 2PDT LATCHING SW- VERT PCB |
| 89100042 | SW3 | SW-SPEC12 | 2PDT LATCHING SW- VERT PCB |
| 57000110 | U1 | NJM2082DD | SELECTED DUAL BIFET OPAMP-8 PIN DIP |
| 57000110 | U11 | NJM2082DD | SELECTED DUAL BIFET OPAMP-8 PIN DIP |
| 57000113 | U12 | NJM2114D | DUAL LOW-NOISE OPAMP DIP8 |
| 57000110 | U13 | NJM2082DD | SELECTED DUAL BIFET OPAMP-8 PIN DIP |
| 57000113 | U14 | NJM2114D | DUAL LOW-NOISE OPAMP DIP8 |
| 57000110 | U15 | NJM2082DD | SELECTED DUAL BIFET OPAMP-8 PIN DIP |
| 57000113 | U16 | NJM2114D | DUAL LOW-NOISE OPAMP DIP8 |
| 57000110 | U17 | NJM2082DD | SELECTED DUAL BIFET OPAMP-8 PIN DIP |
| 57000113 | U18 | NJM2114D | DUAL LOW-NOISE OPAMP DIP8 |
| 57000110 | U19 | NJM2082DD | SELECTED DUAL BIFET OPAMP-8 PIN DIP |
| 57000110 | U2 | NJM2082DD | SELECTED DUAL BIFET OPAMP-8 PIN DIP |
| 57000113 | U20 | NJM2114D | DUAL LOW-NOISE OPAMP DIP8 |
| 57000110 | U21 | NJM2082DD | SELECTED DUAL BIFET OPAMP-8 PIN DIP |
| 57000094 | U22 | SSM-2142P | BAL LINE DRIVER- 8 PIN DIP |
| 57000094 | U23 | SSM-2142P | BAL LINE DRIVER- 8 PIN DIP |
| 57000094 | U24 | SSM-2142P | BAL LINE DRIVER- 8 PIN DIP |
| 57000113 | U3 | NJM2114D | DUAL LOW-NOISE OPAMP DIP8 |
| 57000113 | U4 | NJM2114D | DUAL LOW-NOISE OPAMP DIP8 |
| 57000110 | U5 | NJM2082DD | SELECTED DUAL BIFET OPAMP-8 PIN DIP |
| 57000110 | U6 | NJM2082DD | SELECTED DUAL BIFET OPAMP-8 PIN DIP |
| 57000110 | U9 | NJM2082DD | SELECTED DUAL BIFET OPAMP-8 PIN DIP |
| 68000082 | VR1 | FDR-100MM-SLIM | 10KA 100MM SLIM FADER-PCBMOUNT |
| 68000082 | VR2 | FDR-100MM-SLIM | 10KA 100MM SLIM FADER-PCBMOUNT |
| 68000082 | VR3 | FDR-100MM-SLIM | 10KA 100MM SLIM FADER-PCB MOUNT |