Lexicon Inc.

3 Oak Park

Bedford, Ma 01730-1441

Corrections To PCM 90 Service Manuals

Performance Verification / Troubleshooting Sections

To Make it Compatible to New PCM 91

Author: Jim Ciardelli

Date: 04/08/98

Rev 3

Performance Verification section:

I will be pointing out corrections to the PCN 90 document on this corrections document to prevent mark up confusion in the provided manual.

Page 3-2

XLR cables can also be used if ¼ inch cable are not available for hook up in the required equipment section.

The picture of the back panel showing connections to test equipment must be updated to a PCM 91 back panel.

Page 3-5

The following needs to replace the Digital I/O Test at the bottom of the page

Their are 2 tests that will confirm both the S/PDIF and AES digital in and output ports of the PCM-91.

The first test will test the S/PDIF inputs and outputs of the PCM-91

S/PDIF Test : The PCM-91 has an on board wrap around test in its Extended Diagnostics  which will be used to test the S/PDIF circuit.

- Connect one end of a RCA cable to the S/PDIF IN jack and the other end to the S/PDIF OUT jack.

- Press and hold down the Control and Tempo buttons on the front panel and power up the PCM-91.

- Continue to hold down the buttons until the display reads.

PCM-91 Diagnostics

SELECT Test

- Turn the Select Knob until the display reads.

Dig Aud IO Wrap Test

- Press the Load button the lower have of the display will then read.

WAIT! Executing Test

- If the test is passes it will display PASSED on the display. If it fails the

display  will read FAILURE.

AES Test : The PCM-91 does not have a on board wrap around test in the Extended Diagnostic for the AES circuitry . There for the following can be used to test the ports and circuit of the AES.

It will be necessary to use another PCM-91 or a device that can transmit and receive a AES Signal.

To Keep it simple the test below will be using another PCM0-81.

- Place the two PCM-91s one on top of the other. Connect power to each and power on.

- Attach 2 XLR cables between the AES inputs and outputs of the unit of both PCM-91’s

- On the PCM-91 you are testing , press Control and go to the

Audio*Word  Clock

- Analog: 44.1 kHz  display

- Turn the Adjust knob until the display reads

Audio*Word Clock

- Ext:XLR Prf 44.1

This indicates the PCM-91 in receiving a 44.1 digital signal from the other PCM-91. If how ever it was not the message would read

Audio* Word Clock

0.0 Ext: XLR   No Lock

- To Test the 48kHz  turn the Adjust knob on the other unit to 48.kHz

The display of the unit under test will read

Error:Lock

(Press any button)

- Press a button on the unit and it if the PCM-91 locks on the signal it will the

display will read

Audio*Word Clock

0.0 Ext:XLR Prf 48

Troubleshooting Section:

Page 4-24

Following is a rewrite of the Power Supply section for troubleshooting.

Power Supply

- Plug in the PCM-91 and set the variac for nominal line voltage. A clicking noise from the relays should occur 30 seconds after you have powered up the PCM-91.

- Set DMM to measure VCD, and check the regulated voltages for proper levels For the a ground reference use the ground test point on the board. It is located to the right of U9 labeled GND-4. Below are the test points for the supplies in the unit.

Supplies			Location				Range

+5VD				U37 Pin 20				(4.85-5.35)

+15V				Right of U9				(15.00-16.95)

-15V				Just above U9			(14.25-15.75)

+5VA				Right of U9				(4.75-5.25)

-5VA				Right of U9				(4.75-5.25)

Page 4-25

The battery voltage test must be reworded as follows

- Turn the PCM-91 off and detach the power cord

- Set DMM for a DC voltage reading on the 20 volts DC scale.

- Place the positive Probe (red) of the DMM on top of the battery. Located just behind the front panel on the main board.

- Place the negative probe (black) on the test point marked FOOTPEDAL ADC GND -10 just right of U37 in the middle of the board.

- The reading on the DMM should be ³ 2.75 volts.

- Replace the battery if the voltage is at 2.75 volts or lower.