OSRAM VALVES

| Type Range Valve Fil. Selector Degree B21 2.0 6 16.5 B21 2.0 7 (2nd P.) 16.5 DHD 12.5 8TA do. do. 1DA Max do. do. 5DA Max DHX 12.5 1 6 DL 12.5 1 DPT 12.5 1 8.75 DS 12.5 1 8.5 DSB 12.5 1 6.75 GU1 4.0 1 11 H2 2.0 1 8 H210 2.0 1 21 HL2 2.0 1 8.25 HL210 2.0 1 11 L21 2.0 1 9.5 L210 2.0 1 16 LP2 2.0 1 LS6A 5.0 1 MH4 4.0 1 5.5 MH41 4.0 1 4.25 MH4/K/M 4.0 1 6.5 MHD4 4.0 8TA 6.5 do. do. 1DA Max do. do. 5DA Max MHL4 4.0 1 7.5 MPT4 4.0 1 5.75 MPT4 4.0 6 (7 Pin) 7.5 MPT4/K 4.0 1 7.5 MS4 4.0 1 7.5 MS4B 4.0 1 MSP4 4.0 6 MU12 4.0 1 6.5 do. do. 2 (2nd P.) 6.5 MU14 4.0 1 6.5 do. do. 2 (2nd P.) 6.5 MS4B/K/M 4.0 1 6.5 P2 2.0 1 9.25 P215 2.0 1 13.5 PT2 2.0 1 8.75 PT4 2.0 1 PT16 4.0 1 PT25 4.0 1 PX4 4.0 1 8.75 PX25 4.0 1 S21 2.0 1 11.5 S22 2.0 1 7 S23 2.0 1 12 U10 4.0 1 11 do. do. 2 (2nd P.) 11 U12 4.0 1 10 do. do. 2 (2nd P.) 10 U14 4.0 1 11 do. do. 2 (2nd P.) 11 VDS 12.5 1 13.5 VDSB 12.5 1 VMP4 4.0 6 | Type Range Valve Fil. Selector Degree VMS4 4.0 1 VMS4B 4.0 1 VMS4/K/M 4.0 1 7.5 VP21 2.0 6 VS21 2.0 1 8.25 TA – Triode Anode DA – Diode Anode |
| --- | --- |

METHODS OF VALVE TESTING

Just what constitutes the best  method  of testing a valve to determine its general  condition,  has  been  the  subject of much discussion among radio engineers.

Opinions  at  one time  were  divided  between the relative efficiency of the  four existing methods  viz:   The true mutual conductance, the power output, the  grid  shift  and the emission test.

An  avalanche of  new valves  settled  the  controversy by bringing the emission tester into  general  favor as a commercial valve tester  because,  it  was  the only  system which could be adapted to incorporate all the following points.

1. Ability to cope  with  the  hundreds of existing valves as well as future           		releases.

2. Simplicity and quickness of operation.

3. Reasonably low selling prices.

4. Practical accuracy.

The true mutual conductance tester, whilst the most accurate, is purely a laboratory instrument  requiring  as many controls and meters as there are  elements  in  a  valve.   Therefore it can be regarded as impracticable for commercial purposes.

The limitation of the so-called "grid shift" and power output systems, combined with  the  fact  that  a well designed emission tester may be  the more  accurate,  has  brought  the latter into universal use.  It will be realised  that the only thing likely to happen to a radio  valve  in  service, (apart from leakage between elements) is a  depreciation  of  the  emitting  qualities  of the cathode element.  So why not test a tube for its emission current?

This has been confirmed by our  own and overseas laboratory comparisons.

When the emission type  valve  test  was first developed, the mistake was made of applying a high voltage to the valve through a voltage dropping resistor.   Consequently  the  voltage applied to the valve (tested as a diode) depended on the current drain.  This meant that a poor valve which had  a low emission, was tested at a higher potential than  a  good  one  and  that true differences of merit could not be arrived at.   Then again there arose the danger of harming a poorish valve through excessive voltage being applied to the grid and injuring the cathode coating.

In the  "Palec"  valve  tester,  a  constant  low voltage is applied and the current  drain  limited  to well below saturation point.

This low voltage is not sufficient to ionize the gas if present in a valve and produce extra emission.  Consequently the “Palec” valve tester is a true emission tester and shows the true condition of the cathode independent of whether gas is present or not.

With regard to a gas test, it must be remembered that the percentage of gassy valves is low among new valves, as the manufacturers are usually careful that no such valves leave the factory.

The percentage of valves which become gassy with use is extremely low, less than ¼ %  actually.

Before a valve becomes gassy, it must usually, be seriously overloaded.  This overloading causes a drop in the emission capabilities of the cathode.  This condition is detected by our tester.

Thus, the tester does, in almost every case, class a gassy tube as poor on account of it usually showing poor emission under the test applied.

# A.C MODEL

## INSTRUCTIONS FOR USE

The operation of completely testing a valve on the PATON “PALEC” A.C. Model Valve Tester should be carried out in the following order:-

No. 1-ADJUST LINE VOLTAGE.

Switch on power and press red button at left bottom marked  “LINE CHECK” The Meter Needle should swing right over the designated line on Dial. Should the needle be above or below this point, adjust to same by means of the top left rotary switch, marked “LINE ADJUST.”   It is important to constantly check the line voltage.  This test is made before plugging in valve.

No. 2 - Select correct filament voltage as shown on chart.

No. 3 - Plug in valve.

No. 4 - Test for inter-element shorts and leaks: While the valve is heating, throw “ELEMENT SHORTS” toggle switch to “ON” and rotate the selector switch from point round the full semi-circle.

A shorted element or leakage even as high as 500,000 Ohms will cause the Neon                	electrodes to give off a distinct and definite glow.   (This can he demonstrated by 	connecting a half meg pig tail resistor between  two of the valve pins and noting the 	intensity of this glow).   When the valve is fully heated a very faint glimmer may be       	noticed when  on the Cathode position, but this could denote a resistance leakage of                       	some 	millions of Ohms and can safely be disregarded.

If the valve passes the above test proceed as follows:

No. 5 - Throw Element Shorts toggle to “OFF”

No. 6 - Turn Selector Switch to the point as shown on chart.

No. 7 - Adjust Range Degree control also as per chart.

No. 8 - Press green button marked “MERIT TEST” and the meter will show directly the 	   condition of the valve.

In the cases of Cathode type Valves allow plenty of time for the Valve to become               	fully heated.  This will he shown when the Meter needle stops creeping and remains                          	stationary.

N B.-The Meter Dial is calibrated to show the percentage efficiency of a Valve.

A variation of 10% or so may he noted in new Valves.  This is quite normal.

DIODE PLATES - A comparative test can he made on the Diode Plates by turning  	Range Control to Maximum positi6n (unless otherwise stated)  and Selector Switch to 	the respective point shown on chart, marked DA.

The readings of both plates should be noted between the forty per cent and sixty per                    	cent margin on Meter Dial, and should be approximately the same.

To Identify the Shorting or Leaking Elements.

The points of the Selector Switch correspond to the various elements of a Valve and are numbered according to 1935 standard practice, with the exception that the valve cap is numbered 8.

No.2 is the plate position of all American Valves, while No. 5 is the general  Cathode position.

Filament pins 3 and 4 are not represented on switch, being unnecessary.  A glow at any one point denotes a short to filament, while a glow at any two points shows  the short to be between the two elements indicated.

### MULLARD’S VALVES

| Type Range Valve Fil. Selector Degree ACO44 4.0 11 12.75 ACO64 4.0 1 14.5 ACI04 4.0 1 10 D026 4.0 1 11 DU2 4.0 1 10.0 do. do. 8 (2nd P.) 10 DW3 4.0 1 do. do. 2 (2nd P.) DW4 4.0 1 10.0 do. do. 2 (2nd P.) DW15 7.5 1 9.5 DW15 7.5 2 (2nd P.) 9.5 FC4 4.0 6 6.5 IW3 4.0 1 do. do. 2 (2nd P.) MM4V 4.0 1 6.5 Pen. 4VA 4.0 6 6.5 PM1A 2.0 1 18.5 PM1DG 2.0 1 18 PM1HF 2.0 1 PM1HL 2.0 1 17.5 PM1LF 2.0 1 PM12 2.0 1 PM12A 2.0 1 8.25 PM12M 2.0 1 8.5 PM12V 2.0 1 14.25 PM14 4.0 1 18.5 PM16 6.3 1 20.0 PM2 2.0 1 PM2A 2.0 1 8.25 PM2B 2.0 1 8.0 do. do. 6 8.0 PM2DX 2.0 1 13.0 PM3 4.0 1 12 PM4 4.0 1 14.5 PM4DX 4.0 1 9.5 PM5V 6.0 1 PM5X 6.0 1 15 PM6 6.0 1 12.5 PM22A 2.0 1 8 PM24 4.0 1 12.5 PM24A 4.0 1 10.5 PM24B 4.0 1 8.25 PM24M 4.0 1 10.5 PM26 6.0 1 11.5 PM202 2.0 1 10.5 PM252 2.2 1 PM256 6.0 1 PM2BA 2.0 1 9.5 do. do. 6 (2nd P.) 9.5 SP2 2.0 6 7 SP4 4.0 6 5.5 S4V 4.0 1 12.5 S4VA 4.0 1 4.5 S4VB 4.0 1 TDD2 2.0 8TA 12.25 do. 2.0 1DA Max | Type Range Valve Fil. Selector Degree do. do. 6DA Max TDD4 4.0 8TA 9 do. do. 5DA 11 do. do. 7DA 11 VM4V 4.0 6 VP2 2.0 6 6.25 VP4 4.0 6 5.5 104V 4.0 1 6.5 164V 4.0 1 9.25 244V 4.0 1 5.75 354V 4.0 1 6 904V 4.0 1 5.5 TA – Triode Anode DA – Diode Anode |
| --- | --- |

PHILIPS’ VALVES

| Type Range Valve Fil. Selector Degree A209 2.0 1 17 A409 4.0 1 14.5 A415 4.0 1 10.5 A425 4.0 1 12.75 A435 4.0 1 A442 4.0 1 18.5 A609 5.0 1 13.5 # A615 5.0 1 10.25 # A630 5.0 1 12 # A635 5.0 1 20.5 # A642 5.0 1 13.25 # ABC1 4.0v 8TA 11 # do. do. 7DA Max # do. do. 6DA Max # AC2 4.0v 8 6.5 # AF2 4.0v 6 5.5 # AF3 4.0v 8 7.5 # AF7 4.0v 8 7.5 # AK1 4.0 6 6 # AK2 4.0 7 7.5 # AL2 4.0 8 8.0 # AL3 4.0 7 5.25 # AZ3 4.0 2 6.0 # do. do. 6 (2nd P.) 6.0 # B217 2.0 1 17 # B240 2.0 1 8 # do. do. 6 (2nd P.) 8 # B242 2.0 1 # B255 2.0 1 12.25 # B262 2.0 1 8.75 # B403 4.0 1 19 # B405 4.0 1 13.5 # B406 4.0 1 18 # B409 4.0 1 10 # B443 4.0 1 13 # B605 5.0 1 17 # C234N 2.0 1 9.25 # C443 4.0 1 14.25 # C603 5.0 1 15 # C643 5.0 1 15 # CB1 12.5 1 # do. do. 8 (2nd P.) # CBC1 12.5 8TA 13.0 # do. do. 6DA Max # CBC1 do. 7DA Max # CC1 12.5 8 6.75 # CF1 12.5 8 7.25 # CF2 12.5 8 7 # CK1 12.5 7 7.5 # CL2 25.0 8 6 # CL4 30.0 8 6.0 # CY2 30.0 2 5.25 # do. do. 2 (2nd P.) 5.25 # D243 2.5 1 11.5 # E406 4.0 1 10.5 # E409 4.0 1 6.5 # E424 4.0 1 6 # E438 4.0 1 | Type Range Valve Fil. Selector Degree E442 4.0 1 9.5 E442S 4.0 1 11.5 E443H 4.0 1 10 E443N 4.0 1 13.5 E444 4.0 6 5.5 do. do. 1DA Max E445 4.0 1 9.5 E446 4.0 6 5.5 E447 4.0 6 5.5 E452T 4.0 1 4.75 E454 4.0 8TA 8.25 do. 4.0 1DA Max do. do. 6DA Max E455 4.0 1 5 E463 4.0 6 7.25 EBC1 6.3 8TA 8.5 do. do. 7DA Max do. do. 6DA Max EBF1 6.3 8TA 18.0 do. do. 1DA Max do. do. 6DA Max EB4 6.3 1DA 8.5 do. do. 6DA 8.5 E460N 4.0 7 13.0 EF6 6.3 8 7.25 EF5 6.3 8 7.25 EK2 6.3 7 9.0 EL5 6.3 7 5.5 EL2 6.3 8 6.75 EL3 6.3 7 5.25 EZ2 6.3 2 8.25 do. do. 6 8.25 EZ3 6.3 2 6.0 do. do. 6 6.0 EZ4 6.3 2 5.0 do. do. 6 5.0 F4434.0 4.0 1 12.5 F443N 4.0 1 11.5 KBC1 2.0 8TA 12.5 do. do. 1DA Max do. do. 6DA Max KC3 2.0 7 6.75 KDD1 2.0 6 10 do. do. 1 (2nd P.) 10 KF1 2.0 6 6.75 KF2 2.0 6 7.25 KF3 2.0 8 14.0 KF4 2.0 8 12.5 KK2 2.0 7 15.0 KL4 2.0 7 10.0 506 4.0 1 11 do. do. 2 (2nd P.) 11 1561 4.0 1 8 do. do. 2 (2nd P.) 8 1867 4.0 1 9 do. do. 2 (2nd P.) 9 TA -- Triode Anode DA – Diode Anode |
| --- | --- |

D.C. MODEL

FOR TESTING BATTERY TYPE VALVES

INSTRUCTIONS FOR OPERATION

No. 1-Adjust filament voltage switch to correct voltage stud, (See chart)

No. 2-Plug in valve and test for shorts by throwing bottom right-hand switch to. the “Shorts and Ohms” position.  Rotate selector switch, watching the meter needle for any deflection.   If a leakage is shown the degree of same can be read on Ohmmeter Scale.   The needle going over the full scale indicates a direct short circuit between two or more elements.   (See Par., To identify the shorting of leakage of elements)

No. 3-Adjust “B.” battery voltage by throwing bottom right-hand switch to “B. bat. check position.   Adjust to full scale by means of bottom left  knob marked “B. bat. adj.”

No. 4-Turn selector and range switches to positions as shown on chart.

No. 5-Press button marked “Merit test” and read the  condition  of the valve directly on the good-bad ,scale.

EXTERNAL VOLTAGE READJNG.

The D.C. Model incorporates three ranges of external volts for checking all types of batteries.   Throw bottom right-hand switch to “External Volts” and plug test leads into Wands Sockets at top of panel, selecting the required range.

EXTERNAL OHMS.

A means of checking resistors or condensers for leakage is available by  turning bottom right-hand switch to “Short and Ohms” position. into sockets marked Ext. Ohms.   Read resistance value on ohm meter scale.

AMERICAN VALVES

| Type Range Valve Fil. Selector Degree 01A 5.0 1 18 1A4 2.0 8 21.0 1A6 2.0 6 22 1B4 2.0 8 21.0 1B5 2.0 5 17.3 1C4 2.0 8 11.5 1C6 2.0 6 18 1D4 2.0 1 9.5 1F4 2.0 1 12.5 1F6 2.0 8TA 21.0 1F6 2.0 6DA Max 1F6 2.0 5DA Max 1V 6.3 2 6 2A3 2.5 1 9 2A5 2.5 6 11 2A6 2.5 8TA 9 2A6 2.5 1DA Max 2A6 2.5 6DA Max 2A7 2.5 6 8.75 2B7 2.5 8TA 17 2B7 2.5 7DA Max 2B7 2.5 6DA Max 5W4 5.0 5 3.5 5Z3 5.0 2 7 do. do 1 (2nd P.) 5Z4 5.0 2 6.0 5Z4 5.0 1 (2nd P.) 6.0 6A4 6.3 1 9.75 6A6 6.3 1 do. do 6 (2nd P.) 6A7 6.3 6 8.75 6A8 6.3 1 9.5 6B7 6.3 8TA 16 do. do. 7DA Max do. do. 6DA Max 6C5 6.3 1 9.75 6C6 6.3 8 9 6D5 6.3 1 6D6 6.3 8 10 6F5 6.3 8 7.0 6F6 6.3 1 12.5 6F7 6.3 8 17.5 do. do 8 (2nd p.) 6H6 6.3 1 6.5 do. do. 2 (2nd P.) 6.5 6J7 6.3 8 10.5 6K7 6.3 8 11.5 6L6 6.3 1 7.25 6L7 6.3 8 7.5 6N7 6.3 1 10.0 6N7 6.3 6 (2nd P.) 10.0 6Q7 6.3 8TA 10.0 6Q7 6.3 1DA Max 6Q7 6.3 6DA Max 6R7 6.3 8TA 11.0 6R7 6.3 1DA Max 6R7 6.3 6DA Max # 6X5 6.3 1 7.25 | Type Range Valve Fil. Selector Degree 10 7.5 1 18 12A 5.0 1 11.5 12Z3 12.5 2 5 15 2.5 8 5 19 2.0 1 14.75 do. do. 6 (2nd P.) 14.75 20 3.3 1 24A 2.5 8 10 25A6 25.0 1 9.5 25S 2.0 5 17.25 25Z5 25.0 2 5 do. do. 5 (2nd P.) 5 25Z6 25.0 1 6.0 26 1.5 1 16 27 2.5 1 12.5 30 2.0 1 16.5 31 2.0 1 23 32 2.0 8 19 33 2.0 1 13 34 2.0 8 17 35 2.5 8 10 36 6.3 8 10 37 6.3 1 10 38 6.3 8 11 39 6.3 8 8 40 41 6.3 6 10 42 6.3 6 10 43 25.0 6 7.75 44 6.3 8 8 45 2.5 1 15 46 2.5 1 13 47 2.5 1 11 48 30.0 6 49 2.0 1 17.5 50 7.5 1 21 53 2.5 1 9.5 do. do. 6 (2nd P.) 9.5 55 2.4 8TA 17 do. do. 1DA Max do. do. 6DA Max 56 2.5 1 11.5 57 2.5 8 9 58 2.5 8 9 59 2.5 7 11.5 71A 5.0 1 16.5 75 6.3 8TA 8.5 do. do. 1DA Max do. do. 6DA Max 76 6.3 1 11 77 6.3 1 9.5 78 6.3 8 10.5 79 6.3 1 8.75 do. do. 8 (2nd P.) 8.75 80 5.0 2 11 do. do. 1 (2nd P.) 11 TA – Triode Anode DA – Diode Anode |
| --- | --- |

AMERICAN VALVES

| Type Range Valve Fil. Selector Degree 81 7.5 2 11.5 82 2.5 2 14.5 do. do. 1 (2nd P.) 14.5 83 5.0 2 12.0 do. do. 1 (2nd P.) 12.0 84 6.3 2 6 do. do. 1 (2nd P.) 6 85 6.3 8TA 18 do. do. 1DA Max do. do. 6DA Max 89 6.3 8 12 | Type Range Valve Fil. Selector Degree TA – Triode Anode #### DA – Diode Anode |
| --- | --- |